WHAT ARE HACKATHONS?

VIRTUAL ENVIRONMENT LAB

Setup and configuration

I.T SUPPORT  VERSION 1.2 (28.05.21)

Andy Cooper

East Norfolk Multi Academy Trust

Author Name – 00/00/0000

Supporting information – this should

be no more than three lines.

Author Name – 00/00/0000

contents

brief	2

2. host server specification and configuration	2

2.1 Physical Configuration	2

2.2 ESXI Environment Base configuration	3

2.3 ESXI Roles and Permissions	3

2.3.1 Custom roles	4

3. Pre-built VM Configuration	4

3.1 Firewall	4

3.2.1 Purpose of each network	6

3.2.2 Firewall rules	6

3.2 Windows 2019 Server Standard	10

3.3 Windows 10 Professional	11

4.1.2 Services Section	12

4.1.3 Guardian Section	12

4.2.1 Interfaces	13

4.2.2 System Menu -> Routing	13

# brief

To create a virtual environment to complement other tools such as Cisco Packet Tracer where students are exposed to the creation and configuration of virtual machines covering multiple operating systems.

Certain aspects of the environment will be required to access external services using a number of otherwise restricted ports.

The Environment to be configured in such a way to prevent unauthorized access attempts to the existing college network and servers.

# 2. host server specification and configuration

## 2.1 Physical Configuration

| Host Server specification and configuration |  |
| --- | --- |
| Brand/Model | Dell R730 |
| Processor Type: | 2 x Xeon CPU E5-2660 V2 2.6GHz |
| Sockets: | 2 |
| Cores per socket: | 10 |
| Logical Processors: | 40 |
| RAM: | 256GB |
| Physical Drives: | 2 x 360 SAS Raid 1 Mirror + 1 HS 2 x 1.92TB SSD SATA (VM Storage) |
| IP Address: | 10.11.0.10 |
| Subnet: | 255.255.0.0 |
| Gateway: | 10.11.0.254 |
| DNS Host | Vmlab.eastnorfolk.ac.uk |
| Host OS: | ESXI 7.0 Community Edition |
| Username: | Root |
| Password: |  |

## 2.2 ESXI Environment Base configuration

| Port Group | VLAN ID | Virtual Switch | Network Range |
| --- | --- | --- | --- |
| VM Network | 0 | vSwitch0 | 10.11.0.0/16 |
| PG Internal Switch 1 | 1000 | Internal Switch 1 | 10.20.0.0/16 |
| PG Internal Switch 2 | 1001 | Internal Switch 2 | 10.30.0.0/16 |
| PG Internal Switch 3 | 1002 | Internal Switch 3 | 10.40.0.0/16 |

## 2.3 ESXI Roles and Permissions

| Users | Role |
| --- | --- |
| root | Administrator |
| acooper | Teacher |
| Mrowe | Teacher |
| nwhite | Teacher |

## 2.3.1 Custom roles

# 3. Pre-built VM Configuration

## 3.1 Firewall

The firewall is a critical component of the configuration and must not be deleted. The firewall is responsible for routing and controlling traffic flow between the virtual network switches and subnets. The responsible parties for the virtual lab environment have access to add firewall rules to govern what traffic is and isn’t allowed between the subnets.

| VM Name: | vPFSense |  |
| --- | --- | --- |
| OS: | FreeBSD 12 or later |  |
| Username: | Admin |  |
| Password: | Epsilon_Delta-52 |  |
| VM Function: | Firewall |  |
| IP Address: | 10.11.0.1 (WAN) [VMX1] 10.20.0.254 (LAN) [VMX1] 10.30.0.254 (LAN) [VMX2] 10.40.0.254 (LAN) [VMX3] | Port Groups VMNetwork PG Internal Switch1 PG Internal Switch2 PG Internal Switch3 |
| Gateway: | 10.11.0.254 |  |
| DNS: | 172.22.1.1 | Configured from DNS Resolver and System General Setup Menu |
| vCPU: | 2 |  |
| Disk Size: | 8GB |  |
| Disk Type: | Thin Provision |  |
| Memory: | 1GB |  |
| Network Cards: | 4 VMX0 – Internal Network 4 VMX1 - WAN VMX2 – Internal Network 1 VMX3 – Internal Network 3 |  |
| Port Group Connection: | VMNetwork PG Internal Switch1 PG Internal Switch2 PG Internal Switch3 |  |
| DHCP: | Internal Network 1 Internal Network 2 Internal Network 3 | Range: 10.20.0.11 – 10.20.0.50 Range: 10.30.0.11 – 10.30.0.50 Range: 10.40.0.11 – 10.40.0.50 |
| Static ranges available | Internal Network 1 Internal Network 2 Internal Network 3 | Range: 10.20.0.51 – 10.20.255.254 Range: 10.30.0.51 – 10.30.255.254 Range: 10.40.0.51 – 10.40.255.254 |

The upstream firewall connection (also a PFSense server [PFS01]) is made via the WAN interface. The responsible parties for the virtual lab environment do not have access to these rules or configuration. The traffic has been designed for the following:

No access from any of the VM lab networks onto the main college network subnets.

Traffic to the clouded is all directed through Smoothwall.

DNS Forwarding is taken from the virtual lab environment PFSense server and forwarded directly to the College’s external DNS server located in the DMZ

HTTP, HTTPS traffic is allowed to the cloud via Smoothwall from ‘Internal Network 1’ [10.20.0.0/16).

HTTP, HTTPS traffic and any ports to the cloud via Smoothwall from ‘Internal Network 2’ [10.30.0.0/16]

No network traffic is permitted to the cloud from ‘Internal Network 3 [10.40.0.0/16]’

In order to manage and access the virtual machines from the classroom in the virtual lab, environment traffic has been configured on PFS01 to:

Allow RDP (3389) and (SSH 22) access to ‘Internal Network 1’ and ‘Internal Network 2’ and ‘Internal Network 3’

## 3.2.1 Purpose of each network

Internal Network 1

For use in the creation of server/client configurations. For example, domain controller with the allocation of group policies etc.

Internal Network 2

For use in the creation of various VMs and operating systems (Windows and Linux etc.). Firewall configuration allows for penetration testing.

Internal Network 3

For use in the creation of various VMs. Has no external network access and can be configured for testing traffic through the firewall i.e. configure VPN scenarios.

The responsible parties for the virtual lab environment have the ability to control any traffic between the interfaces.

WARNING – DO NOT ADD ANY ADDIONAL NETWORK CARDS TO THE VM AS THIS WILL RE-ARRANGE THE NETWORK DESIGNATIONS IN PFSENSE

## 3.2.2 Firewall rules

| INTERFACE: WAN | Description:; Allow traffic from main network to VM Internal 1. Action:; Pass. Protocol:; Any. Port:; Any. Source Network:; 10.1.0.0/16. Destination Network:; 10.20.0.0/16. Description:; Allow traffic from main network to VM Internal 2. Action:; Pass. Protocol:; Any. Port:; Any. Source Network:; 10.1.0.0/16. Destination Network:; 10.30.0.0/16. Description:; Allow traffic from main network to VM Internal 3. Action:; Pass. Protocol:; Any. Port:; Any. Source Network:; 10.1.0.0/16. Destination Network:; 10.40.0.0/16. Description:; Allow https traffic from main network to firewall. Action:; Pass. Protocol:; TCP. Port:; HTTPS (443). Source Network:; 10.1.0.0/16. Destination Network:; 10.11.0.1. |
| --- | --- |
| INTERFACE: INTERNAL 1 | Description:; Anti-lockout rule (default). Action:; Pass. Protocol:; Any. Port:; 80 443. Source Network:; Any. Destination Network:; INTERNAL 1 Address (10.20.0.254). Description:; Block bogon networks. Action:; Block. Protocol:; Any. Port:; Any. Source Network:; Reserved Not assigned by IANA. Destination Network:; Any. Description:; Block traffic to internal 2. Action:; Block. Protocol:; Any. Port:; Any. Source Network:; 10.20.0.0/16. Destination Network:; 10.30.0.0/16. Description:; Block traffic to internal 3. Action:; Block. Protocol:; Any. Port:; Any. Source Network:; 10.20.0.0/16. Destination Network:; 10.40.0.0/16. Description:; Enable HTTP to external. Action:; Pass. Protocol:; TCP/UDP. Port:; HTTP (80). Source Network:; Any. Destination Network:; Any. Description:; Enable HTTPS to external. Action:; Pass. Protocol:; TCP/UDP. Port:; HTTPS (443). Source Network:; Any. Destination Network:; Any. Description:; Enable DNS requests to interface. Action:; Pass. Protocol:; TCP/UDP. Port:; DNS (53). Source Network:; 10.20.0.0/16. Destination Network:; 10.20.0.254. |
| INTERFACE: INTERNAL2 | Description:; Block bogon networks. Action:; Block. Protocol:; Any. Port:; Any. Source Network:; Reserved Not assigned by IANA. Destination Network:; Any. Description:; Block traffic to internal 1. Action:; Block. Protocol:; Any. Port:; Any. Source Network:; 10.30.0.0/16. Destination Network:; 10.20.0.0/16. Description:; Block traffic to internal 3. Action:; Block. Protocol:; Any. Port:; Any. Source Network:; 10.30.0.0/16. Destination Network:; 10.40.0.0/16. Description:; Allow all to external interface. Action:; Pass. Protocol:; Any. Port:; Andy. Source Network:; 10.30.0.0/16. Destination Network:; Any. |
| INTERFACE: INTERNAL3NOINTERNET | Description:; Block bogon networks. Action:; Block. Protocol:; Any. Port:; Any. Source Network:; Reserved Not assigned by IANA. Destination Network:; Any. |

## 3.2 Windows 2019 Server Standard

| VM Name: | PermServer2019 |  |
| --- | --- | --- |
| OS: | Windows 2019 Standard |  |
| Username: | Administrator |  |
| Password: | Password1 |  |
| VM Function: | Domain controller |  |
| IP Address: | 10.20.0.1 | Connected to PG Internal Switch 1 |
| Gateway: | 10.20.0.254 |  |
| DNS: | 127.0.0.1 | DNS server configured and forwards to interface 10.20.0.254 for DNS forwarding |
| vCPU: | 2 |  |
| Disk Size: | 60GB |  |
| Disk Type: | Thin Provision |  |
| Memory: | 4GB |  |
| Network Cards: | 1 |  |
| Port Group Connection: | PG Internal Network 1 |  |

## 3.3 Windows 10 Professional

Currently as a standalone PC, not connected to the virtual lab domain. This VM is connected to PG Internal Switch 2 (10.30.0.0/16).

| VM Name: | Win10 |  |
| --- | --- | --- |
| OS: | Windows 10 Professional |  |
| Username: | Administrator |  |
| Password: | Password1 |  |
| VM Function: | Workstation |  |
| IP Address: | DHCP assigned | Connected to PG Internal Switch 1 |
| Gateway: | 10.30.0.254 |  |
| DNS: | 10.30.0.254 | DNS server configured and forwards to |
| vCPU: | 2 |  |
| Disk Size: | 48GB |  |
| Disk Type: | Thin Provision |  |
| Memory: | 4GB |  |
| Network Cards: | 1 |  |
| Port Group Connection: | PG Internal Network 1 |  |

# 4. Modifications to existing core college systems

4.1 Smoothwall

4.1.1 Network section

| Address Object Created: | VMLabs Internal 1 Network (IP Range: 10.20.0.1 – 10.20.0.100) VMLabs Internal 2 Network (IP Range: 10.30.0.1 – 10.30.0.100) |
| --- | --- |
| Locations Object Created: | VM Lab Machine Exceptions (IP Ranges 10.20.0.1 – 10.20.0.100, 10.30.0.1 – 10.30.0.100) |
| Routing Subnets Added: | 10.11.0.0.to GW 10.7.0.1 10.20.0.0 to GW 10.7.0.1 10.30.0.0 to GW 10.7.0.1 |
| Firewall Rule Added: | VM Internal Network 2 to all external interfaces allowing all ports. |

## 4.1.2 Services Section

| Group Created: | VMLab |
| --- | --- |

## 4.1.3 Guardian Section

| HTTPS Inspection Policy Added: | Who:; Everyone Unauthenticated Ips. What:; All Categories All HTTPS Content. Where:; VM Lab Machine Exceptions. When:; Always. Action:; Validate certificate only. |
| --- | --- |
| Web Filtering Policy Added: | Who:; Everyone Unauthenticated Ips. What:; All Categories Everything. Where:; VM Lab Machine Exceptions. When:; Always. Action:; Allow. |

4.1.4 Web Proxy Section

| Authentication Policy Added (1.2) | Interface:; Internal. Method:; No Authentication. Behaviour:; Allow Transparent HTTPS incompatible sites. Where:; VM Lab Machine Exceptions. Unauthenticated requests:; VMLab. |
| --- | --- |

## 4.2.1 Interfaces

| Interface Added: | VMLABNETWORK |
| --- | --- |

## 4.2.2 System Menu -> Routing

| Gateway Added: | Name:; TrafficVMInternal1. Interface:; VMLABNETWORK. Gateway:; 10.11.0.1. Monitor IP:; 10.11.0.1. Description:; Informs main network of location of VM Labs network. |
| --- | --- |
| Static Routes Added: | Network:; 10.20.0.0/16. Gateway:; TrafficVMInternal1. Interface:; VMLABNETWORK. Description; Traffic to VM Lab Internal 1. Network:; 10.30.0.0/16. Gateway:; TrafficVMInternal1. Interface:; VMLABNETWORK. Description; Traffic to VM Lab Internal 2. Network:; 10.40.0.0/16. Gateway:; TrafficVMInternal1. Interface:; VMLABNETWORK. Description; Traffic to VM Lab Internal 3. |

4.2.3 Firewall Rules

| INTERFACE: MAIN | Description:; Allow RDP to Internal Network 1 (10.20.0.0). Action:; Pass. Protocol:; TCP. Port:; RDP (3389). Source Network:; 10.1.0.0/16. Destination Network:; 10.20.0.0/16. Description:; Allow RDP to Internal Network 2 (10.30.0.0). Action:; Pass. Protocol:; TCP. Port:; RDP (3389). Source Network:; 10.1.0.0/16. Destination Network:; 10.30.0.0/16. Description:; Allow RDP to Internal Network 3 (10.40.0.0). Action:; Pass. Protocol:; TCP. Port:; RDP (3389). Source Network:; 10.1.0.0/16. Destination Network:; 10.40.0.0/16. Description:; Allow SSH to Internal Network 1 (10.20.0.0). Action:; Pass. Protocol:; TCP. Port:; SSH (22). Source Network:; 10.1.0.0/16. Destination Network:; 10.20.0.0/16. Description:; Allow SSH to Internal Network 2 (10.20.0.0). Action:; Pass. Protocol:; TCP. Port:; SSH (22). Source Network:; 10.1.0.0/16. Destination Network:; 10.30.0.0/16. Description:; Allow SSH to Internal Network 3 (10.20.0.0). Action:; Pass. Protocol:; TCP. Port:; SSH (22). Source Network:; 10.1.0.0/16. Destination Network:; 10.40.0.0/16. |
| --- | --- |

| INTERFACE: VMLABNETWORK | Description:; Allow All Traffic from VM Internal Network 2 (10.30.0.0). Action:; Pass. Protocol:; Any. Port:; Any. Source Network:; 10.30.0.0/16. Destination Network:; 10.7.0.0/16. Description:; Allow HTTP Traffic from VM Internal Network 1 (10.20.0.0). Action:; Pass. Protocol:; TCP. Port:; HTTP (80). Source Network:; 10.20.0.0/16. Destination Network:; 10.7.0.0/16. Description:; Allow RDP to Internal Network 3 (10.40.0.0). Action:; Pass. Protocol:; TCP. Port:; HTTPS (443). Source Network:; 10.20.0.0/16. Destination Network:; 10.7.0.0/16. Description:; Allow DNS Traffic Virtual PFSense (10.11.0.1). Action:; Pass. Protocol:; TCP/UDP. Port:; DNS (53). Source Network:; 10.11.0.1. Destination Network:; 172.22.1.1 (Ext DNS Server). |
| --- | --- |

THANK YOU

Our Partners							Funded by