Working with biohazards

Hazard perception – Laboratory hazards – Food safety and quality control

Speciation reference A4.5–A4.8

Define the term biohazard.

A microorganism, cell culture or human endoparasite, whether genetically modified or not, which may cause infection, allergy, toxicity or otherwise create a hazard to human health.

Match the hazard groups with the biohazards (biological agents) and examples.

- Can cause severe human disease and may be a serious hazard to employees; it may spread to the community, but there is usually effective prophylaxis or treatment available.

- Causes severe human disease and is a serious hazard to employees; it is likely to spread to the community, and there is usually no effective prophylaxis or treatment available.

- Can cause human disease and may be a hazard to employees; it is unlikely to spread to the community, and there is usually effective prophylaxis or treatment available.

- Unlikely to cause human disease.

St John Rigby College is delivering this programme on behalf of the Education and Training Foundation. This programme is funded by the Department for Education.

Mycobacterium tuberculosis, influenza types A, B and C, Congo haemorrhagic fever virus and Bacillus subtilis

| Hazard group | Letter | Example |
| --- | --- | --- |
| 1 | d | Bacillus subtilis |
| 2 | c | Influenza types A, B and C |
| 3 | a | Mycobacterium tuberculosis |
| 4 | b | Congo haemorrhagic fever virus |

Read the background information below, observe the technicians, complete the proforma below and watch the video (please note this is a fictional scenario for educational purposes).

Background information

This laboratory handles group 3 biological agents. The room has a self-closing door which isolates it from the rest of the building, but technicians revealed that in the summer they have this door propped open to cool the laboratory. The input air and extract air are high-efficiency particulate absorbing (HEPA) filtered, but it was noted that the filter is currently out of compliance as it has not been changed in six months. On inspection, there were no disinfection or incidence logs kept for the laboratory. The laboratory has a high turnover of staff, but all members of staff in the video are authorised to work in this area. Waste material contaminated with viable microorganisms is autoclaved before being disposed of.

St John Rigby College is delivering this programme on behalf of the Education and Training Foundation. This programme is funded by the Department for Education.

Complete the table below

| Hazard no | Hazard – Video still (when appropriate) | Identify the breach(es) of the Control of Substances Hazardous to Health Regulations (2002) (COSHH) | Outline the possible implications of COSHH breaches | Identify the correct containment measures which should be used here |
| --- | --- | --- | --- | --- |
| 1 |  | Unsafe storage of biological agents. | Risks to employee health (short- and long-term effects of infection) Risks to the wider population (disease spread) Risks to the environment (vegetation, water supply and soil) | Biohazardous waste should be treated and disposed of promptly and not allowed to accumulate. Each container of untreated biohazardous waste must be clearly identified as such and labelled with the biohazard symbol. Non-biohazardous waste should be clearly labelled to prevent commingling. Biohazardous material should be placed in an autoclave bag and sterilised prior to removal. |
| 2 |  | Personnel should wear protective clothing. | Risks to employee health (short- and long-term effects of infection) | Comply with any provision in the Personal Protective Equipment (PPE) Regulations (2002). Laboratory staff must wear PPE when conducting work in the laboratory. PPE must be removed on leaving the laboratory, and hygiene practices including hand washing must be rigorously maintained. PPE could include disposable gloves and should include a laboratory coat or gown as a minimum. It may also include eye protection and other equipment as identified by risk assessment. |
| 3 |  | The workplace needs to be separated from any other activities in the same building such as eating, drinking and socialising. | Risks to the wider population (disease spread), water supply and soil | Safe work practices and policies must be in place to reduce the risk of exposure to staff while working in the laboratories. Safe work practices include no eating, drinking, chewing gum, handling of personal electronic devices, smoking, applying cosmetics or handling of contact lenses All PPE must be removed prior to leaving the laboratories, ie lab coats and gloves should never be worn to the bathroom or to get a drink from the water fountain. All staff and learners must wash their hands immediately if contaminated with potentially infected material. Prior to leaving the laboratories handwashing sinks are to be used for handwashing only. No other materials may be poured into these sinks. |
| 4 |  | Infected material not handled in a safety cabinet, isolator or other suitable containment. | Risks to employee health (short- and long-term effects of infection) Risks to the wider population (disease spread) Risks to the environment (vegetation, water supply and soil) | Agar plates, universals and other vessels containing liquid cultures must be stored securely in such a way as they are not in danger of being spilt or knocked over. This can be achieved by: Storing in trays Use of suitable tube racks Ensuring cultures / plates are not near the edge of benches or in areas where there is a danger of spillage. A minor spillage should be handled by applying disinfectant to the spillage and leaving for an appropriate period. The spillage and disinfectant should then be mopped up with disposable paper towels or spillage absorption granules, which should be discarded as clinical waste. For larger liquid spills, it may be appropriate to contain the spill before applying disinfectant. If there has been significant skin contamination, it should be washed off with water, or an emergency shower used if available. Personnel dealing with the spillage should wear the appropriate PPE while doing so. |
| 5 | Filter not changed | Input air and extract air to the workplace are to be filtered using HEPA or equivalent. | Risks to the wider population (disease spread) | Standard operating procedures (SOPs) should be in place and the HEPA filter changed as per the manufacture guidance and monitored via a log. |
| 6 | No log | Specified disinfection procedure. | Risks to employee health (short- and long-term effects of infection) Risks to the wider population (disease spread) Risks to the environment (vegetation, water supply and soil) | SOP to ensure disinfection procedure, ie before and after each procedure by trained technician, including recording the disinfectants used, rotation and batch number. |
| 7 |  | Viable microorganisms should be contained in a system which physically separates the process from the environment (closed system). | Risks to employee health (short- and long-term effects of infection) Risks to the wider population (disease spread) Risks to the environment (vegetation, water supply and soil) | Safe work practices and policies must be in place to reduce the risk of exposure to the physical environment outside of the laboratory. |
| 8 | Door open | Access is to be restricted to authorised persons only. | Risks to the wider population (disease spread) | Door should remain closed and clear signage should be displayed on it regarding access being restricted to authorised personal only. |
| 9 |  | Not complying with specific waste disposal regulations. | Risks to the wider population (disease spread) | Biohazardous waste should be treated and disposed of promptly and not allowed to accumulate. Each container of untreated biohazardous waste must be clearly identified as such and labelled with the biohazard symbol. Non-hazardous waste should be kept separate from hazardous wastes, ie the container shown is for contaminated sharps only. |

St John Rigby College is delivering this programme on behalf of the Education and Training Foundation. This programme is funded by the Department for Education.