Trip project outputs

Introduction to engineering mechanics

Hugh Baird College

May 2023

et-foundation.co.uk

T Level resource-improvement projects (TRIPs)

Final outputs: A seven-week course to support the understanding of science concepts in the T Level core content for the Engineering and Manufacturing route.

Project title: Introduction to engineering mechanics

Organisation: Hugh Baird College, Merseyside

Project finish date: May 2023

Description of outputs: The project comprises a rationale for the approach, a scheme of work, seven three-hour sessions with detailed lesson plans, worksheets, learner self-assessment forms and seven sets of PowerPoint slides. Each session includes a presession task on which the lesson is developed.

Creation process: Each lesson was created to build on the preceding session with assumptions made about learner prior learning as detailed in session 1. Each session is a combination of underpinning scientific knowledge based on Newton’s laws followed by problem-based learning activities to show how the skills, knowledge and understanding would be applied in a workplace context. Other relevant underpinning skills are embedded in each session, and these are identified in the introduction to the session. The sessions have been designed so that organisations who do not have three-hour sessions can easily adapt to their timetable.

Sector use: The course content is relevant for any T Level from the Engineering and Manufacturing Route.

Transferability: These concepts are relevant for all occupational specialisms of the three pathways listed below:

- T Level Technical Qualification in Maintenance, Installation and Repair for Engineering and Manufacturing (8730-12)

- T Level Technical Qualification in Engineering, Manufacturing, Processing and Control (8730-13)

- T Level Technical Qualification in Design and Development for Engineering and Manufacturing (8730-14).

This programme is funded by

the Department for Education

Funded by

Produced

by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Rationale for the course –

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Rationale for the course

This document explains the rationale for the design of this seven-week course to support the understanding of science concepts in the T Level Technical Qualification in Engineering and Manufacturing (Core) (8730).

Throughout this series of sessions, a problem-based learning (PBL) approach that explores an automated overhead crane system has been chosen. This seven-week course covers the following science concepts:

- Newton’s three laws of motion, including appropriate calculations

- understanding and skills involving units of measurement used in engineering

- understanding and skills in vectors applications

- understanding of scientific methods and approaches to scientific inquiry and research

- apply measurement equipment, techniques and principles.

All teaching and learning resources have been developed using the underpinning knowledge outcomes (Core) listed in City & Guilds specifications of three pathways listed below:

- T Level Technical Qualification in Maintenance, Installation and Repair for project Engineering and Manufacturing (8730-12)

- T Level Technical Qualification in Engineering, Manufacturing, Processing and Control (8730-13)

- T Level Technical Qualification in Design and Development for Engineering and Manufacturing (8730-14).

Each session will start with a recap of prior learning followed by problem-based learning activities to situate the learning in real-life context.

This is followed by teaching and learning scientific concepts as follows:

- free body diagrams and equilibrium analysis techniques

- types of forces on simply supported (SS) beams

- support reactions of SS beams

- shear force and bending moment analysis of SS beams

- principle of conservation of momentum

- energy and power in mechanical systems

- types of power sources.

Each session will embed the development of skills, knowledge and understanding which naturally occur in science, maths, English, digital, employability, health and safety, examination and the employer-set project as relevant.

Learners are assumed to have certain level of prior learning, and this is highlighted in each session. This will come from bespoke diagnostic assessments (not provided), but each session will include elicitation and pre-teach as a fail-safe for learners whose knowledge is incomplete or dormant.

The course has adopted a problem-based learning approach to develop learners’ skills that can be used when completing the employer-set project. Sessions have been designed to be active and engaging, with the use of experiential learning methods. Videos (You-tube clips), online experiment simulation tools, in-class experiments, etc. have been embedded in the sessions. Teachers have the flexibility to use any similar physical experimental set-ups instead of the online simulation tools, depending on the availability of the apparatus.

Each session includes at least two opportunities for formative assessments for the whole class. In addition, at the end of each session, learners will self-assess their progress. In between sessions, learners will be expected to complete a pre-session task as part of their development of employability skills.

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Glossary of terms and equations sheet

Application of Newton’s laws

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Glossary of terms

Contact forces: Force exerted between two objects when they are touching.

Deceleration: Slowing down or negative acceleration, e.g., the car slowed down with a deceleration of 2 ms−².

Friction: A force that opposes or prevents movement and converts kinetic energy into heat.

Inertia: The tendency of an object to continue in its current state (at rest or in uniform motion) unless acted on by a resultant force.

Inertial mass: A measure of how difficult it is to change the velocity of an object. It is defined as the ratio of force over acceleration.

Kinetic energy: Energy which an object possesses by being in motion.

Magnitude: The size of a physical quantity.

Mass: The amount of matter an object contains. Mass is measured in kilograms (kg) or grams (g).

Momentum: A quantity relating to a moving object that is calculated by multiplying its mass by its velocity.

Non-contact Forces: The push or pull acting between objects that are not physically touching when they interact.

Normal force: This is the component of a contact force that is perpendicular to the surface that an object is in contact.

Resultant force: The single force that could replace all the forces acting on an object, found by adding these together. If all the forces are balanced, the resultant force is zero.

Terminal velocity: The maximum speed of an object, reached when the forces moving the object are balanced by its frictional forces.

Thrust: A force used to move a body forward or up, e.g., the rocket had a thrust of 10,000 N.

Tractive force: The force needed to overcome the resistance caused by friction when two bodies slide or roll on each other.

Vector: A vector describes a movement from one point to another. A vector quantity has magnitude (size) and direction.

Velocity: The speed of an object in a particular direction.

Work done: The amount of energy it takes to do a task. Measured in joules (J). For example, the work done in raising a mass through 10 m would be equal to the gain in potential energy of the mass.

## Equations for sessions

### Newton's second law of motion

Resultant force = mass × acceleration

F=ma

This is when:

- F is force, measured in newtons (N)

- m is mass, measured in kilograms (kg)

- a is acceleration, measured in metres per second squared (m/s²).

### Calculating weight

The equation to calculate weight in newtons is:

Weight = mass × gravitational field strength

W=mg

Where:

- W is weight, measured in newtons (N)

- m is mass, measured in kilograms (kg)

- gravitational field strength (g) is 9.81 N/kg (on the Earth’s surface).

### Static equilibrium

Fx=0 , Fy=0, M=0

Where:

- F is force, measured in newtons (N)

- M is moment, measured in kilograms (Nm).

### Tractive force

F= μTW

= μTmaG

Where:

- F is traction effort or force acting on the wheel from the surface, measured in newtons (N)

- μt is the traction – or friction – coefficient between the wheel and the surface

- W is weight or vertical force between wheel and surface, measured in Newtons (N)

- M is mass on the wheel, measured in kilograms (kg)

- ag is acceleration due to gravity and is 9.81 m/s2.

### Kinetic energy (KE)

KE=12mv2

Where:

- m is mass, measured in kilograms (kg)

- v is speed or velocity, measured in metres per second (m/s)

- KE is kinetic energy, measured in Joules (J).

### Gravitational potential energy (GPE)

GPE=mgh

Where:

- GPE is gravitational potential energy, measured in joules (J)

- m is mass, measured in kilograms (kg)

- g is gravitational field strength, measured in newtons per kilogram (N/kg)

- h is height, measured in metres (m).

### Elastic potential energy (EPE)

EPE=12kx2

Where:

- EPE is measured in joules (J)

- k is the spring constant, measured in (N/m)

- x is extension, measured in metres (m).

### Power (P)

P=Et

Where:

- P is power, measured in watts (W)

- E is work done, measured in joules (J)

- t is time, measured in seconds (s).

### Equations of motion

v=u+at

s=ut+12at2

v2=u2+2as

s=12u+vt

Where:

- t is time, measured in seconds (s)

- u is velocity at t=0, measured in metres per second (m/s)

- v is velocity at t=t, measured in metres per second (m/s)

- a is acceleration in time, t, duration, measured in metres per second squared (m/s2)

- s is displacement in time, t, duration, measured in metres (m).

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Scheme of work

etfoundation.co.uk

Introduction to engineering mechanics

| # Seven-week course on introduction to engineering mechanics T Level route: Engineering and Manufacturing T Level pathway: T Level Technical Qualification in Design and Development for Engineering and Manufacturing City & Guilds number: 8730 |  |  |
| --- | --- | --- |
| Curriculum intent This seven-week course is designed to support the understanding of the application of science concepts of the T Level core content in the industrial settings of the Engineering and Manufacturing Route. A problem-based learning (PBL) approach allows learners to explore an automated overhead crane system in this series of sessions. The PBL approach helps to develop learners’ skills that can be used when completing the employer-set project. This course covers the following science concepts: - Newton’s three laws of motion, including appropriate calculations (5.6, 7.1, 7.2) *City & Guilds specification numbers - understanding and skills of units of measurement used in engineering (5.1)* - understanding and skills in vectors applications (5.2)* - understanding of scientific methods and approaches to scientific inquiry and research (5.3)* - apply measurement equipment, techniques and principles (5.4)* These concepts are relevant for the Mechanical, Electrical and Electronic, Control and Instrumentation and Structural Engineering occupational specialisms. Each session will start with a recap of prior learning, followed by problem-based learning activities to situate the learning in a real-life industrial context. Each session will embed the development of science, maths, English, digital, employability, health and safety, examination and the employer-set project as relevant. Each session assumes a level of prior learning, sourced from bespoke diagnostic assessments (not provided), however each session will include elicitation and pre-teach as a fail-safe for learners whose knowledge is incomplete or dormant. |  |  |
| Prior learning Maths diagnostic assessment (or pre-teaching) has shown that learners have skills, knowledge and understanding of: - the use of the four rules of number, BIDMAS (brackets, indices, division, multiplication, addition, and subtraction), decimals, fractions, numbers in standard form, linear equations, length, area and 2D volume. Science diagnostic assessment (or pre-teaching) has shown that the learners have skills, knowledge and understanding of: - what force, moment, motion and base units are - notations and symbols used in free body diagrams (FBD) (eg, Mg, R, W, F and N). |  |  |
| Overview of assessment Each session will include at least two opportunities for formative assessments for the whole class. In addition, at the end of each session, learners will self-assess their progress. In between sessions, learners will be expected to complete a pre-session task. This introduces learning for the next session and by completing the task learners will be developing employability skills such as enthusiasm, preparedness and independence. |  |  |
| Sequence of learning |  |
| 1 | - Identify a range of situations where simply supported (SS) beams are applied in the industry. - Identify structural elements of static and dynamic systems. - Construct FBD. - Calculate forces in static systems using Newton’s first and third laws. - Calculate forces and accelerations using Newton’s second law. |
| 2 | - Understand the types of forces (concurrent, non-concurrent, coplanar, non-contact). - Understand the loading types on SS beams. - Understand types of supports used in SS beams. - Perform an experiment on SS beams and record results. |

| 3 | - Calculate the support reactions on SS beams subjected to: - point loads - uniformly distributed loads (UDL) - combination of both point and UDLs. - Compare support reaction of SS beams from experiments and theoretical values. |
| --- | --- |
| 4 | - Calculate shear force of SS beams. - Calculate bending moment analysis of SS beams. - Model SS beams using beam analysis software/platforms. |
| 5 | - Calculate momentum of moving objects. - Understand principle of conservation of momentum. - Calculate velocity or mass using conservation of momentum. |
| 6 | - Understand different types of mechanical energy. - Calculate mechanical energy for given systems. - Calculate power in mechanical systems. |
| 7 | - Identify types of power sources. - Understand the need for renewable energy sources. - Understand sustainable materials used to reduce energy requirement. |

Produced by

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

This programme is funded by

the Department for Education

Introduction to engineering mechanics

Lesson plan one

| Session number: 1 of 7 | Engineering mechanics: Application of Newton’s laws |
| --- | --- |
| City & Guilds ref | 8730 |
| Tutor notes and session rationale This session will prepare the learners for the second session, in which the types of forces on simply supported (SS) beams are analysed. The session starts with an introduction to the lesson and the course which is assessed using Geoff Petty’s true/false formative assessment activity (see slides 8 and 32). This is to establish a continuum of formative assessment needed in examination-based courses. Learners prior knowledge: Teaching is started with elicit/pre-teach activities to ensure all learners’ knowledge of the following is sound and has not become dormant. Maths diagnostic assessment (or pre-teaching) has shown that learners have skills, knowledge and understanding of: - the use of the four rules of number, BIDMAS, decimals, fractions, numbers in standard form, linear equations, length, area and 2D volume. Science diagnostic assessment (or pre-teaching) has shown that the learners have skills, knowledge and understanding of: - what force, motion and base units are - notations and symbols used in free body diagrams (FBD) (eg, Mg, R, W, F, and N). |  |  |
| Embedded skills development |  |  |
| Science | Application, knowledge and understanding of Newton’s three laws of motion, including appropriate calculations. (5.6, 7.1 and 7.2) *City & Guilds specification numbers Understanding and skills of units of measurement used in engineering. (5.1)* Understanding and skills in vectors applications. (5.2)* Understand scientific methods and approaches to scientific inquiry and research. (5.3)* Apply measurement equipment, techniques and principles. (5.4)* |
| Maths | Apply linear equations to calculate forces and accelerations in practical application of Newton’s laws. |
| English | Develop note-taking skills. Summarise information and ideas. |
| Digital | An introduction to how digital skills such as presentations and use of CAD will be developed in this course. |
| Employability | The problem-based learning (PBL) project includes opportunities to develop problem-solving skills that are used in industry. |
| Health and safety | Highlight the health and safety aspects in relation to design of this structure |
| Employer-set project | An introduction to how the skills developed through PBL activities can be applied to the employer-set project. |
| Examinations | The science concepts that will be assessed as part of the core content for the T Level in Engineering and Manufacturing. Guidance on how to read examination questions carefully. |

| Outcomes – by the end of the session learners will: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Understand the content of this course and start to be able to generalise the science concepts to a range of topics in the Engineering and Manufacturing route. Demonstrate the ability to: - identify a range of situations where SS beams are applied in the industry - identify structural elements of static and dynamic systems - construct FBD - calculate forces in static systems using Newton’s first and third laws. - calculate forces and accelerations using Newton’s second law |  |  |  |  |  |
| Time | Tutor activity | Learner activity |  | Resources and slide number | Assessment of learning |  |  |
| 15 mins | Introduce the session and overall course content, aims and objectives. Ask the learners to make notes on their copy of the slides. Emphasise that note taking is a key skill they will use throughout the course, in the employer-set project and in future occupation. Discuss suitable applications of such systems in engineering and manufacturing. | Listen to the presentation, make notes on a copy of slides. and ask questions for clarification if necessary. |  | PowerPoint soft copy and hard copy: Section 01, Slides 1 to 8 | Three true/false assessment questions |  |  |
| 20 mins | Introduce the PBL for this course. Elicit or pre-teach the physical elements that transfer forces in a structure using the room and building they are currently in. Introduce the pre-video question on slide 11. Put the learners in pairs or small groups. Show the videos of the design project. Probe learner responses for deeper understanding of the applications on slide 12 and 13. Collate learner answers on whiteboard. | Answer questions and make notes. Share ideas on the pre-video question. Watch the videos and make notes. Discuss answer with a colleague and offer answers. Photograph or note the collated answers. |  | PowerPoint soft copy and hard copy: Section 02, Slides 9 to 13 2 videos | Probing questions |  |  |
| 40 mins | Explain the process of constructing an FBD using the video link. Ask learners to list the steps to construct FBD. Lead learners to self-assess their work. Elicit or pre-teach key terms and concepts of examples of how to construct an FBD, including Mg and N, R, W, F. Model the FBD examples in slides 17, 18 and 19. Ask learners to construct the FBD and identify the types of forces acting on the identified structural elements. Mentor the learners as they work. Lead learners to self-assess their work. Lead a discussion on where this skill can be used elsewhere in the T Level. | Follow the explanation on video and list the steps to construct FBD. Compare answers with the sample steps of FBD on slide 16. Study the examples of FBD on slides and follow explanations. Seek clarification if necessary. Make note on steps and construct FBDs. Compare answers with the FBDs on slides 17, 18 and 20. Learners each construct the FBD. Identify the type of forces acting on the systems. Seek clarification if necessary. Compare answers with slide 22 and teacher’s answer on whiteboard. Offer examples of where it might be used elsewhere. |  | PowerPoint soft copy and hard copy: Section 03 Video link for FBD A4 sheets to construct FBD/ pencils/ ruler. Slides 17 to 20 Glossary of terms for the course Slides 21 and 22 Whiteboard to model the examples. The complete FBD is given as a separate teaching resource. | Construction of FBDs |  |  |
| 15 mins | Break |
| 25 mins | Teach the principles required for the calculation of Newton's three laws. Model involving algebraic equation solving: - For the first law: use of a learner to model the stationary object under different forces. Use of planetary motion to further model the situation. - Second law: use two learners to model the imbalanced force situation. Use of a free-falling object to further model the situation. - Third law: use a bird flapping its wings in mid-flight to model the reaction force and effects. Use of the same concept in jet engines to model the reaction force and its effects. Encourage a discussion of possible application of Newton’s laws, probe questions to enhance understanding. | Follow the explanation. Learners to participate in the modelling of balanced/unbalanced forces as guided by the teacher. Make notes on the models. Answer the probing questions and seek clarification if necessary. Offer examples of where Newton’s laws might be used. | PowerPoint soft copy and hard copy: Section 04, Slides 23 to 32 Whiteboard to model the examples. |  | Probing questions Three true/false assessment questions at the end |  |

| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| --- | --- | --- | --- | --- |
| 20 mins | Provide time for sample questions and practice activities on slides 33 and 34. Do the working out on the whiteboard. Act as scribe for sample answers. Summarise the approach for problem solving | Calculate the examples on slides 33 and 34. Seek clarification if necessary. Mark the answer using the teacher’s workings. Make notes on the problem-solving approach. | PowerPoint soft copy and hard copy: Section 04, Slides 33 to 37 Whiteboard to show the workings | Directed questions to explain the calculation |
| 30 mins | Applying the Newton’s laws for industrial question solving: Health and safety aspects and structural design failures. Provide the worksheets and facilitate the calculations. Probe questions to consider health and safety aspects from each design cases detailed in the worksheet. Collect the completed worksheets (mark before prior to next session). | Complete the worksheet and hand-over to teacher. Seek clarification if necessary. Answer the probing questions and make notes. | Own notes Calculator Worksheet in print Glossary of terms and equations sheet in print | Independent worksheet completion Probing questions |
| Stretch and challenge. | Facilitate the FBDs fine-tuning process for the problem-based learning project (PBL) for any learners finishing the worksheets earlier to the given time frame. | Fine tune the FBD of the PBL with the guidance of the tutor | A4 sheets to construct FBD/pencils/ruler |  |
| 15 mins | Plenary Elicit learning from the learners and scribe on board. Give out the pre-session task of session 2 along with the video link of pre-session video. | Learners offer recap of main learning in the session. Gain access to the pre-session video link and task. | Self-assessment sheet Pre-session task Pre-session video link | Elicitation Self-Assessment |

Produced by

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

This programme is funded by

the Department for Education

Session 1 – worksheet

Application of Newton’s laws

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 1 worksheet

### Scenario:

ABC is an automobile manufacturing company that is considering designing and building a double girder automated overhead crane (AOC) system (Figure 01) to assist their car manufacturing process.

Figure 01: Double girder AOC system.

Structural design is the systematic process of creating a safe and functional structure when it is subjected to external loads. There are four mandatory steps to be followed for the design of any structure:

  - modelling

  - load analysis

  - structural analysis

  - structural design and detailing.

The aim is to analyse how the structural model behaves with the different load combinations. This then leads to the selection of the materials and dimensioning of the different elements such as beams, columns, joints, supports, foundations, etc. of the structure based on the analysis results.

- As part of the design process, the engineers have decided to analyse the cable tension for different configurations of the hoist cable attached to the traversing trolley (Figure 02). The results will then be used to select cable materials and dimensions. Figures 03(a) and (b) illustrate the distinctive design options considered for the hoist mechanism of AOC system.

Figure 02: The traversing trolley of the double girder AOC system

- Calculate the tension of cables/ wire ropes, for each of the following cases.

Case 1: Single cable hoist as in (a) with zero weight attached to the hook.

Case 2: Single cable hoist as in (a) with 1 tonne attached to the hook.

Case 3: Double cable hoist as in (b) with zero weight attached to the hook.

Case 4: Double cable hoist as in (b) with 1 tonne attached to the hook.

(a)

(b)

5 kg

5 kg

Figure 03: Two cable hoist cases

- Safe working loads. The term safe working load (SWL) for a wire rope is used to define the load that can be applied to the rope allowing it to provide efficient service and to prolong the life of the rope. Because of the high expected service on the slings (that connect the load to the lifting device), errors in determining load weights, the effect of sling angle on the loading, etc. it is mandatory that all safe working loads be based on a safety factor of at least 5 to 1.

The lead structural engineer has suggested using bright wire, uncoated, fibre core (FC), improved plow steel (IPS) wire rope as the hoist cable material. Based on the calculated tension of the rope of cases 2 and 4 from question 1a, recommend a suitable diameter for each case.

You can use the link below to access the SWL table of the selected material.

https://www.engineeringtoolbox.com/wire-rope-strength-d_1518.html

- The engineers have decided that the traversing trolley design (Figure 02 above) will be finalised based on the case 4 loading conditions that was detailed in question 1a.

This task details the steps associated with calculation of the force required to move the traversing trolley in constant velocity along the girder. This will be used to calculate the amount of work done by the trolley per second, thus select a suitable motor to provide the required force, which will be taught in session 6 and 7 of this course.

Design details: The trolley has a mass of 15 kg. The kinetic friction coefficient between the girder and the trolley has been calculated as 0.73.

- Draw the free body diagram (FBD) of the stationary trolley.

- Calculate the reaction forces acting on the stationary trolley.

- Calculate the frictional force acting on the trolley when the trolley is moving.

- Calculate the force required to move the traversing trolley in constant velocity along the girder.

- The design engineers of ABC company are working on an automated ramp system for their manufacturing workshop. As part of the design process, they consider the following case.

A hollow box will be used to transport the tools to an elevated workshop area as illustrated in figure 04 below. The box will be pulled along an inclined ramp (20° to horizontal) using a pulley-cable system. Both the box and the ramp surface are to be made with a frictionless material. The maximum mass of the box filled with the tools will be approximately 125 kg.

The tasks below detail the steps associated with calculating the tension of the rope when the box moves up the ramp at a constant velocity. This will be used to recommend a suitable diameter for the cable.

- Draw the FBD for the inclined ramp.

- Calculate the value of the tension force when the box is moving at a constant velocity.

- Calculate the value of the normal force.

- Calculate the cable tension, if the kinetic friction coefficient of the surface is 0.42 and the box is moving at constant velocity.

- Recommend a suitable diameter for the cable if the material used is polyester. The SWL table can be accessed through the link below.

https://www.engineeringtoolbox.com/polyester-rope-strength-d_1514.html

Figure 04: Tool box being raised on a ramp

The calculated cable tension in question 3d will be used to calculate the amount of work done by the box per second, thus select a suitable motor to provide the required force. These calculations will be done in session 6 and 7 of this course.

## Pre-session task – session 2 Pre-session video link: How to Calculate Support Reactions of a Simply Supported Beam with a Point Load – YouTube *Please note you may need to verify your age to watch this link The double girder AOC system has two girders to support the traversing trolley mechanism as illustrated in Figures 01 and 02. The design engineers are considering using only one traversing trolley on the girders shown on the drawings in the scenario. One of the design cases for the girders/beams design is explained below. The girders are pin supported at the two ends and the traversing trolley is stationary at the centre of the girder. The trolley has a mass of 15 kg and is not attached to the hoist cable system. The mass of the trolley is equally distributed between the two girders; hence each girder is subjected to only half of the weight of the trolley. The girder length is 10 m and mass is 375 kg. - Draw the FBD for one girder, using the design details given above. - Calculate the reaction forces at the two ends of the girder. - Evaluate the effect of the trolley movement on the end reactions.

## Self-assessment sheet

### Please RAG rate your confidence (Red= I am not confident, Amber = I need more practice, Green = I am confident)

| Learning objectives | R | A | G |
| --- | --- | --- | --- |
| - I know how to construct a free body diagram. | ☐ | ☐ | ☐ |
| - I know how to identify structural elements of static and dynamic systems. | ☐ | ☐ | ☐ |
| - I know how to calculate forces in static systems using Newton's first and third law. | ☐ | ☐ | ☐ |
| - I know how to calculate forces and accelerations using Newton's second law. | ☐ | ☐ | ☐ |
| - I can identify the forces applied in a double girder automated overhead crane (AOC) system. | ☐ | ☐ | ☐ |
| - I understand the application of Newton’s laws in double girder AOC and similar systems. | ☐ | ☐ | ☐ |
| - I know how to access the pre-session video link and the task. | ☐ | ☐ | ☐ |

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Session 1 – worksheet model answers

Application of Newton’s laws

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 1 worksheet – model answers

- The figures below illustrate the different design options considered for the hoist mechanism of AOC system.

- Calculate the tension of cables, for each of the following cases.

Case 1: Single cable hoist as in (a) with zero weight attached to the hook.

Case 2: Single cable hoist as in (a) with 1 tonne attached to the hook.

Case 3: Double cable hoist as in (b) with zero weight attached to the hook.

Case 4: Double cable hoist as in (b) with 1 tonne attached to the hook.

- Case 2: Tension of the rope: 9.86 kN

Material: Improved plow steel (IPS), bright wire, uncoated, fibre core (FC), based on the values in the table, select 9.5mm diameter. This has a minimum breaking strength of 54.3 kN and a safe working load of 10.9 kN (with a 5 : 1 safety factor).

Case 4: Tension of the rope: 4.93 kN

Material: Improved plow steel (IPS), bright wire, uncoated, fibre core (FC), based on the values in the table, select 8 mm diameter. This has a minimum breaking strength of 37.9 kN and a safe working load of 7.56 kN (with a 5 : 1 safety factor).

- The traversing trolley (Figures 02 and 03) design will be finalised using case 4, detailed in the question 1a above. The trolley has a mass of 15 kg. The kinetic friction coefficient between the girder and the trolley has been calculated as 0.73.

- Draw the Free Body Diagram (FBD) of the stationary trolley.

- Calculate the reaction forces acting on the stationary trolley.

- Calculate the frictional force acting on the trolley when the trolley is moving.

- Calculate the force required to move the traversing trolley in constant velocity along the girder.

- The design engineers of ABC company are working on an automated ramp system for their manufacturing workshop. As part of the design process, they consider the following case.

A box made with a frictionless material on a frictionless inclined ramp will be used to transport the tools to an elevated workshop area as illustrated in the below diagram. The maximum mass of the box will be 125 kg.

20˚

Draw the FBD for the inclined ramp.

- Calculate the value of the tension force when the box is moving at a constant velocity.

- Calculate the value of the normal force.

- Calculate the cable tension, if the kinetic friction coefficient of the surface is 0.42 and the box is moving at constant velocity.

20˚

- Recommend a suitable diameter for the cable if the material used is polyester.

Based on the values of the table, select 10 mm diameter. This has a minimum breaking strength of 12.9 kN and a safe working load of 1.07 kN (with a 12 : 1 safety factor).

## Pre-session task – session 2 - Draw the FBD for one girder, using the design details given above. - Calculate the reaction forces at the two ends of the girder. Evaluate the effect of the trolley movement on the end reactions. When the trolley is moving, there will be forces acting along the girder, such as driving force of the trolley and the friction force against the movement. Hence, to balance these, there should be horizontal opposite reactions at the two ends of the girder. This means, the end supports should restrict both vertical and horizontal movements.

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Introduction to engineering mechanics

Lesson plan two

| Session number: 2 of 7 | Engineering mechanics: Application of Newton’s laws |
| --- | --- |
| City & Guilds ref | 8730 |
| Tutor notes and session rationale This session will prepare the learners for the third session, in which the support reactions in simply supported beams are analysed. The session starts with an introduction to the lesson and a recap of session 1 which is then assessed using Geoff Petty’s true/false formative assessment activity (see slide 43). Review of pre-session task is vital in all these sessions, as it establishes the groundwork for the rest of the session. This helps to establish a continuum of self-directed learning and formative assessment needed in examination-based courses. This session assumes that the marked worksheets from session 1 show that the learning has taken place. If the worksheets show there is a significant gap in knowledge from session 1, this will need to be addressed before moving onto session 2. Review of learners’ self-assessment sheets will contribute to tutor’s knowledge of learners’ progress. Learners’ prior knowledge: Teaching of this course is started with elicit/pre-teach activities to ensure all learners’ knowledge of the following is sound and has not become dormant. For more details, refer to lesson plan 1. Resource requirement: Three wooden blocks, two scales, an A4 size book and a metre ruler are required for the experiment detailed in the session 2 worksheet. For more details, refer to experiment worksheet of session 2. Pre-session task submission: Facilitate submission of either soft copy or hard copy of completed pre-session tasks before the session. Maintain a submission record. |  |  |
| Embedded skills development |  |  |
| Science | Application, knowledge and understanding of Newton’s three laws of motion, including appropriate. calculations. (5.6, 7.1 and 7.2) *City & Guilds specification numbers Understanding and skills of units of measurement used in engineering. (5.1)* Understanding and skills in vectors applications. (5.2)* Understand scientific methods and approaches to scientific inquiry and research. (5.3)* Apply measurement equipment, techniques and principles. (5.4)* |
| Maths1 | Interpreting and representing with mathematical diagrams. Using rules and formulae. Communicating using mathematics. |
| English2 | Summarise information and ideas. Develop note taking skills. Take part in/lead discussions |
| Digital3 | Use digital technology and media effectively. Process and analyse numerical data: Use of MS Excel for experiment data presentation and analysis. |
| Employability | The problem-based learning (PBL) project includes opportunities to develop problem-solving skills that are used in industry. |
| Health and safety | Highlight the health and safety aspects in relation to design of this structure. |
| employer-set project | An introduction to how the activities based the learning on this course can be used in an employer-set project. |
| Examinations |  |

| Outcomes – by the end of the session learners will: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Understand the loading arrangements and support reactions on simply supported (SS) beams and start to be able to generalise the SS beams applications to a range of topics in the Engineering and Manufacturing route. Demonstrate the ability to: - understand the types of forces (concurrent, non-concurrent, coplanar, non-contact) - understand the loading types on SS beams - understand types of supports used in SS beams - perform an experiment on SS beams and record results. |  |  |  |  |  |  |  |  |
| Time |  | Tutor activity |  | Learner activity |  | Resources and slide number |  | Assessment of learning |
| 20 mins |  | Discuss the worksheet submission process and provide generic feedback for session 1 worksheets. Briefly elicit the session 1 content covered. Ask learners to offer examples of activities undertaken in session 1 under each objective listed in slide 42. Ask learners to answer three true/false questions, probe questions for deeper understanding. Introduce the session objectives. Ask the learners to make notes on their copy of slides. Check the note-taking method followed by learners and emphasise the importance of note taking as a skill. Demonstration of what an effective note looks like would be ideal. |  | Follow the explanation and make notes. Offer answers about session 1 learning activities. Participate in the discussion and answer three true/false questions. Listen to the presentation, make notes on copy of slides and ask questions for clarification if necessary. Self-evaluation of their note taking method. |  | PowerPoint soft copy and hard copy: Section 01 Glossary of terms Slides 42 to 43 |  | Three true/false assessment questions |
| 15 mins ----------- 15 mins |  | Make small groups and guide them to explore answers to the pre-session video-based question c) which is to evaluate the effect of the trolley movement on the end reactions. Probe learner responses for deeper understanding by revisiting the PBL scenario video on slide 45 and collect answers to whiteboard. ------------------------------------------------------------------- Probe learner groups the presession task a and b, then write responses on whiteboard. Show the answers to pre-session task a, b and c in slide 46 to 49. Lead learners to self-assess their work. Discuss similar applications of such systems in Engineering and Manufacturing. Ask learners to answer three true/false questions, probe questions for deeper understanding. |  | Get into allocated group to discuss, evaluate, summarise and make notes on the answers. Offer ideas, participate in the discussion and make notes. ------------------------------------------------ Learner groups to participate in the presession task answer collection on white board. Self-assess the answers to pre-sessions tasks and make notes. Participate in the discussion. Answer three true/false questions and participate in the discussion. |  | PowerPoint soft copy and hard copy: Section 01 Slides 44 to 50 Videos: - How to Calculate Support Reactions of a Simply Supported Beam with a Point Load – YouTube - (639) EOT Crane ¦ Double Girder ¦ Overhead Crane ¦ EOT Crane Working ¦ How Crane Works? ¦ 3D Animation – YouTube *Please note you may need to verify your age to watch these links Glossary of terms Whiteboard |  | Probing questions three true false assessment questions |
| 40 mins |  | Explain the pre-experiment task. Brainstorm different types of forces and ask learners to watch the video on slide 52 and note the different categories of forces. Ask directed questions to gather answers to the pre-experiment task. Guide learners to self-assess their answers. Ask learners to read the experiment worksheet. Explain the experimental set-up and probe questions on suitable industrial examples for different load arrangements listed in step 4 of the experiment method. Guide learners to place the experimental set-up as detailed in the handout. Guide and supervise learners to follow the steps listed under the method section of the worksheet. Complete steps 1, 2, 3, 4a, 4b and 4c For more understanding of 4a to 4c show slides 55 to 57. Guide learners to complete table A. |  | Offer answers to teacher, follow the explanation on video and answer the pre-experiment question. Offer answers, self-assess and make notes. Read the experiment worksheet, offer answers and participate in the discussion. Follow the guidance of teacher to place the experimental set-up. Participate in the experiment, take the readings for three initial cases listed under method 4. Record results on table A of printed worksheet. |  | PowerPoint soft copy and hard copy: Section 02, Slides 52–57 Video: System of Forces – YouTube *Please note you may need to verify your age to watch this link Glossary of terms Experiment resources, worksheet soft copy and hard copy, pencils/pens |  | Pre-experiment task. Directed questions. Independent experiment results table completion |
| 15 mins |  | Break |  |  |  |  |  |  |
| 40 mins | Guide and supervise learners to continue the experiment from step 4d onwards as per the instructions on the worksheet. For more understanding of 4d to 4g, show slides 58 to 61. Guide learners to complete table A in the printed handout. Use MS Excel for experiment data presentation and analysis. |  | Follow the guidance, participate in the experiment, take the readings for later four cases listed under method 4. Record results in table A of printed worksheet. Enter data on a MS Excel spreadsheet. |  | PowerPoint soft copy and hard copy: Section 02, Slides 58–61 Glossary of terms Experiment resources, worksheet soft copy and hard copy, pencils/pens MS Excel software on computer/laptop |  | Independent experiment results table completion |  |

| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| --- | --- | --- | --- | --- |
| 30 mins | Show slide 63, probe questions to offer examples from the industry or day-to-day life on different types of beams and loadings. Collate answers on whiteboard. Show the videos on slide 63, 64 and 266. Guide learners to take notes. Collate and review answers to the pre-video questions. You can use slide 65 to elaborate on loading function explained in the second video from slide 64. Ask learners to answer three true/false questions, probe questions for deeper understanding. | Offer examples, follow explanation and photograph the collated answers or make notes. Follow the explanation, watch the videos and make notes. Make notes and ask for clarifications if required. Answer three true/false questions and participate in the discussion. | PowerPoint soft copy and hard copy: Section 03, slides 63 to 67 4 videos: - (698) Strength of Materials: Beams – YouTube - (698) Vector Statics: Distributed Forces – YouTube - Point Load vs. Uniform Distributed Load – Federal Brace - Types of Support ¦ Support Reactions in a Beam – YouTube *Please note you may need to verify your age to watch these links Whiteboard to collate answers. | Probing questions three true false assessment questions at the end |
| 15 mins | Plenary Elicit learning related to both session 1 and 2, from the learners and scribe on board. Emphasise the problem-solving steps used in this course. Give out the pre-session task for session 3 along with the video link of pre-session video. Explain the scope of Lifting Equipment Engineers Association (LEEA) and highlight the health and safety aspects from the session. Emphasise pre-requisite knowledge for session 3. Guide learners to complete the self-assessment. | Learners offer recap of main learning in the session. Gain access to the pre-session video link and task. Follow the explanation and make notes. Note the links to access the self-directed revisions. Complete the self-assessment | Whiteboard Pre-session task Pre-session video link: Think Lifting (leeaint.com) Self-assessment sheet | Elicitation Self-assessment |

Produced by

Funded by

This programme is funded by

the Department for Education

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Session 2 – experiment worksheet

Loads and support types of simply supported beams

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 2 Experiment worksheet

Scenario: ABC is an automobile manufacturing company that is considering designing and building a double girder automated overhead crane (AOC) system (Figure 01) to assist their car manufacturing process.

Figure 01: Double girder AOC system.

As part of the design process, the design engineers are analysing the support reactions of the beams.

The aim of the design process is to analyse how the structural model behaves with different load combinations. This leads to selection of the materials and dimensioning of the different elements such as beams, columns, joints, supports, foundations, etc. of the structure based on the analysis results.

A support reaction calculation is an important sub-step of the design process of a beam. We are going to model the AOC girder, using the resources and methods listed below, to observe how the support reactions vary with load and the position of the trolley. The weight imposed by the trolley, or any other object placed on the girder, is simulated using a wooden block and a beam in this experiment.

## Required items:

- two wooden blocks of generally 5 cm of side length

- a wooden/steel beam (you can use a metre ruler)

- two bathroom scales or any similar type of a scale that could be used for this experimental set-up shown in figure 02

- weights such as a wooden block or an A4-sized book.

## Method

- Measure the mass of each wooden block, ruler and book and record in the results table (Table A).

- Measure the length of the book and record in the results table.

- Set-up the beam/ruler as shown in the Figure 02.

Scale A

Scale B

Figure 02: Experimental set-up.

- Record the values shown on the scales A and B in Table A below, for the cases listed below.

- Place the wooden block at the centre of the beam.

- Place the wooden block at a quarter of the beam length, from the left corner of the beam.

- Place the wooden block at a quarter of the beam length, from the right corner of the beam.

- Place the book at the centre of the beam.

- Place the book at the left end of the beam.

- Place the book at the right end of the beam.

- Keep the book at the same place and place the wooden block at the centre of the beam.

Important note: When positioning the wooden block and book, consider the length to the centre of the object.

## Results

### Table A: Results from the experiment

| Mass of the wooden block 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mass of the wooden block 2 |  |  |  |  |  |
| Mass of the wooden block 3 |  |  |  |  |  |
| Mass of the book |  |  |  |  |  |
| Length of the book |  |  |  |  |  |
| Case no. | Scale A reading in kg | Scale B reading in kg |  | Scale A reading in N | Scale B reading in N |
| a |  |  |  |  |  |
| b |  |  |  |  |  |
| c |  |  |  |  |  |
| d |  |  |  |  |  |
| e |  |  |  |  |  |
| f |  |  |  |  |  |
| g |  |  |  |  |  |

Pre-session task for session 3 Task 1: Watch the video given in the session 2 slide 72 and list 10 applications of beams along with a picture (screen-print the objects from the video). Think Lifting (leeaint.com) (It is listed as the first video on this website link, with the title, LEEA released this video to support our Schools Engagement Project.) Example answer: - Wind turbine blades

## Self-assessment sheet

### Please RAG rate your confidence (Red= I am not confident, Amber = I need more practice, Green = I am confident)

| Learning objectives | R | A | G |
| --- | --- | --- | --- |
| - I know the categories of forces such as concurrent, non-concurrent, coplanar, non-contact. | ☐ | ☐ | ☐ |
| - I know how to identify the different loading types on Simply supported (SS) beams such as point loads, uniformly distributed loads and non-uniformly distributed loads. | ☐ | ☐ | ☐ |
| - I understand the application beam analysis in double girder AOC and similar systems. | ☐ | ☐ | ☐ |
| - I know how to access the pre-session video link and understand the task in hand. | ☐ | ☐ | ☐ |

Introduction to engineering mechanics

Lesson plan three

| Session number: 3 of 7 | Engineering mechanics: Application of Newton’s laws |
| --- | --- |
| City & Guilds ref | 8730 |
| Tutor notes and session rationale This session will prepare the learners for the fourth session, in which the shear force and bending moments in simply supported (SS) beams are analysed. The session starts with an introduction to the lesson and a recap of session 2 which is then assessed using Geoff Petty’s true/false formative assessment activity (see slide 80). Reviewing the pre-session task is vital in all these sessions, as it establishes the groundwork for the rest of the session. This helps to establish a continuum of self-directed learning and formative assessment needed in examination-based courses. This session assumes that the completed experimental worksheets have been reviewed by the teacher before session 3. This session is a continuation of the experimental work undertaken at session 2. Learners’ prior knowledge: Teaching of this session starts with eliciting/pre-teaching activities to ensure all learners have understand and can remember this material. For more details, refer to lesson plan 1. Presession task submission: Facilitate submission of either soft copy or hard copy of completed pre-session tasks before the session. Maintain a submission record. |  |  |
| Embedded skills development |  |  |
| Science | Application, knowledge and understanding of Newton’s three laws of motion, including appropriate calculations. (5.6, 7.1 and 7.2). *City & Guilds specification numbers Understanding and skills of units of measurement used in engineering. (5.1)* Understanding and skills in vectors applications. (5.2)* Understand scientific methods and approaches to scientific inquiry and research. (5.3)* Apply measurement equipment, techniques and principles. (5.4)* |
| Maths1 | Interpreting and representing with mathematical diagrams. Using rules and formulae. Communicating using mathematics. |
| English2 | Summarise information and ideas. Develop note-taking skills. Take part in/lead discussions. |
| Digital3 | Use digital technology and media effectively. Process and analyse numerical data: Use of MS Excel for experiment data presentation and analysis. |
| Employability | The problem-based learning (PBL) project includes opportunities to develop problem-solving skills that are used in industry. |
| Health and safety | Highlight the health and safety aspects in relation to design of this structure and beams in general. |
| Employer-set project | An introduction to how the activities and the learning on this course can be used in an employer-set project. |
| Examinations | The science concepts shall be assessed as part of the core content for the T Level in Engineering and Manufacturing. |

| Outcomes – by the end of the session learners will: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Demonstrate the ability to calculate the support reactions on SS beams. Recognise where SS Beams are used in industry. Apply the science concepts to a range of topics found in the industrial environments in the Engineering and Manufacturing route. Demonstrate the ability to: - Calculate the support reactions on SS beams subjected to: - point loads - uniformly distributed loads (UDL) - combination of both point and UDLs. - Compare the support reaction of SS beams from experiments and theoretical values. |  |  |  |  |  |  |  |  |
| Time | Tutor activity |  |  | Learner activity |  | Resources and slide number |  | Assessment of learning |
| 30 mins | Introduce the session objectives. Ask the learners to make notes on their copy of slides. Emphasise that note taking is a key skill they will use throughout the course, in the employer-set project and in future occupation. Briefly explain the content that session 2 covered. Ask learners to offer examples of activities undertaken in session 2 under each objective listed in slide 4. Revisit the experiment steps followed. Ask learners to answer three true/false questions, probe questions for deeper understanding. |  |  | Listen to the presentation, make notes on copy of slides and ask questions for clarification if necessary. Follow the explanation, offer answers about session 2 learning activities. Answer three true/false questions and participate in the discussion. |  | PowerPoint soft copy and hard copy: Section 01 Glossary of terms Slides 79 to 82 |  | three true/false assessment questions |
| 40 mins | Make small groups and guide them to discuss and present answers to the pre-session video-based question. Probe learner responses for a deeper understanding of the applications for beams in the engineering and manufacturing industry and the types of supports used in each case. Ask each learner group to use the whiteboard to model two of the beam applications they suggest. Show selected submissions received for pre-session task and provide generic feedback on the quality of the answers. Ask learners to answer three true/false questions, probe questions for deeper understanding. |  |  | Get into allocated group to discuss, evaluate, summarise and present the answers. Offer ideas, participate in the discussion and make notes. Participate in the modelling of beam applications on the whiteboard. Present ideas to the class. Listen to the explanation and make notes. Answer three true/false questions and participate in the discussion. |  | PowerPoint soft copy and hard copy: Section 01, slides 81 to 82 Glossary of terms |  | Probing questions three true/false assessment questions |
| 20 mins | Using the slide 84, teach the SS beam analysis steps using the slides. Explain the concept of static equilibrium in relation to Newton’s first and second laws. Highlight the difference of static and dynamic equilibrium. Use probe questions to collate industrial examples for the above two equilibrium types. Act as a scribe for sample answers. Model the answer on the whiteboard for case 4a given in slide 85. Model the answer using Beam Calculator Online (Calculate the reactions, Draws Bending Moment, Shear Force, Axial Force) (beamguru.com) Guide learners to enter the results from case 4a in the table B of the worksheet. |  |  | Follow the explanation and make notes. Ask for clarifications if necessary. Offer answers, participate in the discussion and make notes. Follow the manual beam analysis calculation, follow the online simulation and make notes. Record results on the table B of printed worksheet. |  | PowerPoint soft copy and hard copy: Section 02, slides 84 to 11 Glossary of terms and equations sheet Worksheet soft copy and hard copy, pencil/pen, calculator Access to online simulation tool |  | Probing questions Independent experiment results table completion |
| 15 mins | Break |  |  |  |  |  |  |  |
| Time |  | Tutor activity | Learner activity |  | Resources and slide number |  | Assessment of learning |  |
| 30 mins |  | Revisit SS beam analysis steps in slide 84. Explain the cases in section 2 of the slides with the PBL scenario; the weight imposed by the trolley or any other object place on the girder is simulated using a wooden block and a beam. Provide time to calculate support reactions for case 4b and 4c, mentor the learners as they work. Invite a learner to write the answer for each case on the whiteboard and provide feedback for the answers received. Do the working out on the whiteboard if necessary. Use the online simulation tool to solve the same. Lead learners to self-assess their work. Guide learners to complete the results in table B on the worksheet. Emphasise the requirement of support reaction calculation as a step of the design process of a double girder AOC system and ask for different applications from the industry. Guide learners to complete the discussion questions on worksheet, | Follow the explanation and make notes. Calculate the support reactions for case 4b and 4c in the space provided on worksheet. Seek clarification if necessary. Learners offer answers as directed by the teacher, make notes on improving the quality of the answer. Mark the answer using the teacher’s workings. Record results in table B of printed worksheet. Listen to the explanation and offer answers. Make notes and complete the discussion section of the worksheet. |  | PowerPoint soft copy and hard copy: Section 04, slides 84 to 88 Own notes Worksheet soft copy and hard copy, pencil/pen, calculator Whiteboard to show the workings Access to online simulation tool |  | Directed questions to explain the calculation Independent experiment results table completion |  |
| 30 mins |  | Revisit SS beam analysis steps in slide 84. Model the answer on whiteboard for case 4d given in slide 88. Model the answer using online simulation tool. Provide time to calculate support reactions for cases 4e, 4f and 4g. Mentor the learners as they work. Invite a learner to write the answer for each case on the whiteboard and provide feedback for the answers received. Do the workings on the whiteboard if necessary. Use the online simulation tool to solve the same. Lead learners to self-assess their work. Guide learners to complete table B and mentor learners to write answers to the discussion questions on the worksheet. Ask learners to answer three true/false questions, probe questions for deeper understanding. | Follow the explanation and make notes. Calculate the support reactions for cases case 4e, 4f and 4g in the space provided on worksheet. Seek clarification if necessary. Learners offer answers as directed by the teacher, make notes on improving the quality of the answer. Mark the answer using the teacher’s workings. Record results on table B of printed worksheet and answer the discussion questions. Answer three true/false questions and participate in the discussion. |  | PowerPoint soft copy and hard copy: Section 04, slides 84 to 92 Own notes Worksheet soft copy and hard copy, pencil/pen, calculator Whiteboard to show the workings. Access to online simulation tool |  | Directed questions to explain the calculation. Independent experiment results table completion three true/false assessment questions |  |
| 15 mins |  | Plenary Elicit learning from the learners and scribe on board. Probe questions on slide 95 and show the video. Discuss H and S risks associated with poorly designed beams. Give out the pre-session task for next session along with the video link. Provide time to complete self-assessment sheet. | Learners offer recap of main learning in the session. Offer answers, participate in the discussion and make notes. Note the pre-session video link and task. Complete the self-assessment sheet. |  | Video: (698) Bending failure of a softwood beam grain parallel to beam axis: Materials Lab on-line – YouTube *Please note you may need to verify your age to watch this link Pre-session task Pre-session video link on slide 96 Self-assessment sheet |  | Elicitation Probing questions Self-Assessment |  |

This programme is funded by

the Department for Education

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Funded by

Produced by

Session 3 – worksheet

Support reactions on simply supported beams

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 3 worksheet

Scenario: ABC is an automobile manufacturing company that is considering designing and building a double girder automated overhead crane (AOC) system (Figure 01) to assist their car manufacturing process.

Figure 01: Double girder AOC system.

As part of the design process, the design engineers are analysing the support reactions of the beams.

The aim of the design process is to analyse how the structural model behaves with different load combinations. This then leads to selection of the materials and dimensioning of the different elements such as beams, columns, joints, supports, foundations, etc. of the structure based on the analysis results.

We observed support reaction variation on a physically simulated beam set-up in session 2. The weight imposed by the trolley on a double girder AOC system was simulated using a wooden block and a beam in the session 2 experiment.

In today’s session we will be manually calculating the support reactions for the seven cases of load applications on the beam that were tested in the experiment.

## Calculations

Case 4a:

Case 4b:

Case 4c:

Case 4d:

Case 4e:

Case 4f:

Case 4g:

## Results

### Table B: Results from the theoretical calculations

| Case no. | Support reaction at scale A (R1) in N | Support reaction at scale A (R1) in N |
| --- | --- | --- |
| a |  |  |
| b |  |  |
| c |  |  |
| d |  |  |
| e |  |  |
| f |  |  |
| g |  |  |

## Discussion

- Compare your answers from experiments entered in table A, with the theoretically analysed values entered in table B for each experimental case a to g.

- Explain how the support reactions of the two girders shall be used for the design of the double girder AOC system, in the industry.

Pre-session task for session 3 Task: Watch the two videos given below and write the definition and units of: - shearing stress - shear force - bending stress - bending moment. (698) Strength of Materials: Analysis of shearing stress – YouTube (698) Strength of Materials: Bending – YouTube *Please note you may need to verify your age to watch these links

## Self-assessment sheet

### Please RAG rate your confidence (Red= I am not confident, Amber = I need more practice, Green = I am confident)

| Learning objectives | R | A | G |
| --- | --- | --- | --- |
| - I understand the steps to follow for SS beam analysis. | ☐ | ☐ | ☐ |
| - I know how to calculate the support reactions for SS beams subjected to point loads. | ☐ | ☐ | ☐ |
| - I know how to calculate the support reactions for SS beams subjected to uniformly distributed loads. | ☐ | ☐ | ☐ |
| - I understand the application beam analysis in double girder AOC and similar systems. | ☐ | ☐ | ☐ |
| - I know how to access the pre-session video link and understand the task in hand. | ☐ | ☐ | ☐ |

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Introduction to engineering mechanics

Lesson plan four

| Session number: 4 of 7 | Engineering mechanics: Application of Newton’s laws |
| --- | --- |
| City & Guilds ref | 8730 |
| Tutor notes and session rationale This session is focused on calculation of shear force and bending moments in simply supported (SS) beams as part of the structural design process. The session starts with an introduction to the lesson and a recap of session 3, which is then assessed using Geoff Petty’s true/false formative assessment activity. Review of the pre-session task is vital in all these sessions, as it establishes the groundwork for the rest of the session. This helps to establish a continuum of self-directed learning and formative assessment needed in examination-based courses. This session assumes that the completed experimental worksheets have been reviewed by the teacher before session 4. This session is a continuation of the content covered in session 2 and 3. Learners’ prior knowledge: Teaching of this session starts with eliciting/pre-teaching activities to ensure all learners’ knowledge of the following is sound and has not become dormant. For more details, refer to lesson plan 1. Pre-session task submission: Facilitate submission of either soft copy or hard copy of completed pre-session tasks before the session. Maintain a submission record. |  |  |
| Embedded skills development |  |  |
| Science | Application, knowledge and understanding of Newton’s three laws of motion, including appropriate calculations. (5.6, 7.1 and 7.2) *City & Guilds specification numbers Understanding and skills of units of measurement used in engineering. (5.1)* Understanding and skills in vectors applications. (5.2)* Understand scientific methods and approaches to scientific inquiry and research. (5.3)* |
| Maths1 | Interpreting and representing with mathematical diagrams. Using rules and formulae. Communicating using mathematics. |
| English2 | Summarise, synthesise and present information and ideas. Take part in/lead discussions. Develop note taking skills. |
| Digital3 | Use digital technology and media effectively. Communicate and collaborate. Be safe and responsible online. |
| Employability | The problem-based learning (PBL) project includes opportunities to develop problem-solving skills, analytical thinking and teamworking skills that are useful to work in the industry. |
| Health and safety | Highlight the health and safety aspects in relation to design of this structure and beams in general |
| Employer-set project | An introduction to how the activities and the learning on this course can be used in an employer-set project. |
| Examinations | The science concepts shall be assessed as part of the core content for the T Level in Engineering and Manufacturing. |

| Outcomes – by the end of the session learners will: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Apply science concepts to a range of topics found in the industrial environments in the Engineering and Manufacturing Route. Evaluate engineering design failures occurred in the past because of poor structural design practices. Demonstrate the ability to: - calculate shear force of SS beams - calculate bending moment of SS beams - model SS beams using beam analysis software/platforms. |  |  |  |  |
| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| 20 mins | Introduce the session objectives. Explain how the sessions are linked and contribute to the PBL project on automated overhead crane (AOC) design project. Ask the learners to make notes on their copy of slides. Review past session notes of learners and provide feedback for improvement. Briefly explain the session 3 content covered. Ask learners to offer examples of activities undertaken in session 3 under each objective listed in slide 100. Act as scribe for pre-session task answers and review. Explain definitions and sign conventions of shear and bending with industrial examples. Ask learners to answer three true/false questions, probe questions for deeper understanding. | Listen to the presentation, make notes on copy of slides and ask questions for clarification if necessary. Follow the explanation, offer answers about session 3 learning activities. Offer answers to pre-session task. Make notes on definitions and sign conventions. Answer three true/false questions and participate in the discussion. | PowerPoint soft copy and hard copy: Section 01, slides 100 to 108 Glossary of terms | three true/false assessment questions |
| 40 mins | Make small groups and guide them to conduct a case study on failure of structures because of shear and bending stress for one of the topics listed on slide 110. Explain the guidelines on slide 111 to conduct the case study. Facilitate the learner groups to locate suitable resources to conduct case studies on the selected topic and prepare the presentation. Use probing questions for deeper understanding of the design failures to drive the groups in the right direction. Give guidance about the level of work required and show the presentation marking grid. | Get into allocated group and select a project to conduct a case study. Follow the explanation and make notes. Study the resources relevant for the case. Answer questions, delegate responsibilities among team members, analyse design failure data, discuss and summarise the findings in the presentation by studying the marking grid. | PowerPoint soft copy and hard copy: Section 02, slides 109 to 111 Glossary of terms Own notes Laptops/ computers Presentation marking grid | Probing questions PowerPoint slides |
| 30 mins | Facilitate learner groups to present the case study to the class. Encourage learners to have a Q/A round at the end of each presentation. Drive the Q/A round to discuss the design failure-related questions for deeper understanding. Provide generic feedback on the quality of case studies, highlighting any overlooked design features. Ask learners to answer three true/false questions, probe questions for deeper understanding. | Present the case studies to the class. Participate in the Q/A at the end of other presentations. Answer questions at the end of their own presentation. Follow explanation and make notes. Receive feedback and make notes on improving the quality of presentations. Answer three true/false questions and participate in the discussion. | PowerPoint soft copy and hard copy: Section 02 Slides 109 to 111 Glossary of terms Laptops/ computers Presentation marking grid. | Probing questions Presentation questions and answers three true/false assessment questions |
| 15 mins | Break |  |  |  |

| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| --- | --- | --- | --- | --- |
| 30 mins | Using the section 2 – slide 113, teach the SS beam analysis steps to calculate shear and bending stress. Highlight the fact that steps 1 and 2 have been covered in session 3. Explain the PBL design case, in relation to the trolley movement on the girders of AOC using slide 114. Use probing questions to collate industrial examples for similar design situations. Elaborate more on internal stress variations using two videos on slide 115. Model the answer on whiteboard for case 4a given in slides 116 to 118. Model the answer using online beam simulation tool. Probe questions on how the shear force and bending moment diagrams are used in the beams design process. Show the video on slide 119 to illustrate the bending moment variations of a beam. | Follow the explanation and make notes. Ask for clarifications if necessary. Offer answers, participate in the discussion and make notes. Follow the manual beam analysis calculation, follow the online simulation and make notes. Offer answers, watch the video, participate in the discussion and make notes. | PowerPoint soft copy and hard copy: Section 02, slides 113 to 119 Three videos: (7) Bending failure of a softwood beam grain parallel to beam axis: Materials Lab on-line – YouTube Firefighters and Construction: Beams and Girders: How They Fail (fireengineering.com) (24) ANSYS Workbench Tutorial - Simply Supported Beam - Center Load - PART 2 – YouTube A4 sheets/pencil/pen, calculator Access to online simulation tool Beam Calculator Online (Calculate the reactions, Draws Bending Moment, Shear Force, Axial Force) (beamguru.com) *Please note you may need to verify your age to watch these links | Probing questions. Independent experiment results table completion |

| 30 mins | Explain case 4b from the slides, using the PBL scenario; the weight imposed by the trolley or any other object place on the girder is simulated using a wooden block and a beam. Provide time to calculate support reactions for case 4b to 4g, mentor the learners as they work. Invite a learner to write the answer for each case on the whiteboard and provide feedback for the answers received. Do the workings on the whiteboard if necessary. Use the online simulation tool to solve the same. Lead learners to self-assess their work. Guide learners to complete Table B on the worksheet. Emphasise the use of results graphs for the design process of a double girder AOC system and ask directed questions on different applications from the industry. Ask learners to answer three assessment questions on slide 126 and discuss for deeper understanding. | Follow the explanation and make notes. Calculate the support reactions for case 4b and 4c in the space provided on the worksheet. Seek clarification if necessary. Learners offer answers as directed by the teacher, make notes on improving the quality of the answer. Mark the answer using the teacher’s workings. Listen to the explanation, offer answers and make notes. Answer three questions and participate in the discussion. | PowerPoint soft copy and hard copy: Section 04, slides 120 to 126 Own notes Worksheet soft copy and hard copy, pencil/pen, calculator Whiteboard to show the workings Access to online simulation tool | Directed questions to explain the calculation Independent experiment results table completion |
| --- | --- | --- | --- | --- |

| 15 mins | Plenary Elicit learning from the learners and scribe on board. Share the pre-session task on slide 129 and the pre-session video link. Explain the links to momentum principles in trolley movement design in the PBL project. Provide time to complete self-assessment sheet. | Learners offer recap of main learning in the session. Note the pre-session video link and task. Follow the explanation and make notes. Complete the self-assessment sheet. | PowerPoint soft copy and hard copy: Section 04, slides 128 to 131 Video: (7) Applications of Law of Conservation of Momentum (GA_M-NLM22) – YouTube *Please note you may need to verify your age to watch this link Pre-session task Self-assessment sheet | Elicitation Self-Assessment |
| --- | --- | --- | --- | --- |

This programme is funded by

the Department for Education

Produced by

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Session 4 – presentation rubric and self-assessment sheet

etfoundation.co.uk

Introduction to engineering mechanics

Rubric for Presentation (-----/75)

| Category | Scoring criteria | Poor Excellent 1 2 3 4 5 |
| --- | --- | --- |
| Organisation ----- /15 | The presentation is relevant for the topic and audience. | Poor Excellent 1 2 3 4 5 |
|  | A logical sequence is followed to present the content. | Poor Excellent 1 2 3 4 5 |
|  | The references are cited appropriately. | Poor Excellent 1 2 3 4 5 |
| Content -----/25 | Introduction is interesting and establishes a foundation for the rest of the presentation. | Poor Excellent 1 2 3 4 5 |
|  | Technical terms are clearly explained. | Poor Excellent 1 2 3 4 5 |
|  | Presentation contains accurate information and material included is relevant to the overall message/purpose. | Poor Excellent 1 2 3 4 5 |
|  | Appropriate amount of material is prepared and points made reflect well their relative importance. | Poor Excellent 1 2 3 4 5 |
|  | The conclusion is nicely summarising the presentation. | Poor Excellent 1 2 3 4 5 |
| Presentation ----/35 | Speaker uses good gestures and maintains good eye contact with the audience. | Poor Excellent 1 2 3 4 5 |
|  | Speaker uses a clear, audible voice. | Poor Excellent 1 2 3 4 5 |
|  | Delivery is poised, controlled and smooth. | Poor Excellent 1 2 3 4 5 |
|  | Good language skills and pronunciation are used. | Poor Excellent 1 2 3 4 5 |
|  | Visual aids are well prepared, informative, effective and not distracting. | Poor Excellent 1 2 3 4 5 |
|  | Length of presentation is within the assigned time limits. | Poor Excellent 1 2 3 4 5 |
|  | Information was well communicated. | Poor Excellent 1 2 3 4 5 |

Session 4

Self-assessment sheet

Please RAG rate your confidence (Red= I am not confident, Amber = I need more practice, Green = I am confident)

| Learning objectives | R | A | G |
| --- | --- | --- | --- |
| - I understand the steps to follow for SS beam analysis to calculate reaction forces. | ​​☐​ | ​​☐​ | ​​☐​ |
| - I know how to calculate the shear force and bending moment for SS beams subjected to point loads . | ​​☐​ | ​​☐​ | ​​☐​ |
| - I know how to calculate the shear force and bending moment for SS beams subjected to uniformly distributed loads. | ​​☐​ | ​​☐​ | ​​☐​ |
| - I understand the application beam analysis in double girder AOC and similar system. | ​​☐​ | ​​☐​ | ​​☐​ |
| - I know how to access the pre-session video link and understand the task in hand. | ​​☐​ | ​​☐​ | ​​☐​ |

Introduction to engineering mechanics

Lesson plan five

| Session number: 5 of 7 | Engineering mechanics: Application of Newton’s laws |
| --- | --- |
| City & Guilds ref | 8730 |
| Tutor notes and session rationale This session is focused on calculation of impulse, momentum and their applications in the engineering and manufacturing industry. The session starts with an introduction to the lesson and a recap of session 4, which is then assessed using Geoff Petty’s true/false formative assessment activity. Review of the pre-session task is vital in all these sessions, as it establishes the groundwork for the rest of the session. This helps to establish a continuum of self-directed learning and formative assessment needed in examination-based courses. This session assumes that the completed pre-session task and the session 4 self-evaluation have been reviewed by the teacher before session 5. This session uses the same problem-based learning (PBL) project scenario that was used in sessions 1 to 4. Learners’ prior knowledge: Teaching of this session starts with eliciting/pre-teaching activities to ensure all learners’ knowledge of the following is sound and has not become dormant. For more details, refer to lesson plan 1. Pre-session task submission: Facilitate submission of either soft copy or hard copy of completed pre-session tasks before the session. Maintain a submission record. |  |  |
| Embedded skills development |  |  |
| Science | Application, knowledge and understanding of Newton’s three laws of motion, including appropriate calculations. (5.6, 7.1 and 7.2) City & Guilds specification numbers Understanding and skills of units of measurement used in engineering. (5.1)* Understanding and skills in vectors applications. (5.2)* Understand scientific methods and approaches to scientific inquiry and research. (5.3)* |
| Maths1 | Interpreting and representing with mathematical diagrams. Using rules and formulae. Communicating using mathematics. |
| English2 | Summarise, synthesise and present information and ideas. Take part in/lead discussions. Develop note taking skills. |
| Digital3 | Use digital technology and media effectively. Communicate and collaborate. Be safe and responsible online. |
| Employability | The problem-based learning (PBL) project includes opportunities to develop problem-solving skills, analytical thinking and teamworking skills that are useful to work in the industry. |
| Health and safety | Highlight the health and safety aspects in relation to design of the trolley system of an overhead crane system. |
| Employer-set project | An introduction to how the activities and the learning on this course can be used in an employer-set project. |
| Examinations | The science concepts shall be assessed as part of the core content for the T Level in Engineering and Manufacturing. |

| Outcomes – by the end of the session learners will: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apply science concepts to a range of topics found in the industrial environments in the Engineering and Manufacturing route. Evaluate health safety implications of side pulling of cranes and collision of trolleys on overhead cranes. Demonstrate the ability to: - calculate momentum for a given system. - calculate impulse for a given system - calculate velocities using conservation of momentum principle. |  |  |  |  |  |  |  |  |
| Time | Tutor activity |  | Learner activity |  |  | Resources and slide number | Assessment of learning |  |
| 30 mins | Introduce the session objectives. Explain how the sessions are linked and contributing to the PBL project on an automated overhead crane (AOC) design project. Guide learners to make notes on their copy of slides. Ask learners to offer examples of activities undertaken in session 4 under each objective listed in slide 135. Ask learners to answer assessment questions, probe questions for deeper understanding. Act as scribe for pre-session task answers and review. Once all the answers are collected on board, go to slide 138 and play the video which explains the answer. Note that this video is different to the pre-session video. Distribute the pre-marked pre-session task to learners. Explain the answer on whiteboard for more clarity. Probe questions on how the momentum principles shall be used in designing the trolley system of the overhead crane. *This exercise contributes to development of skills required by ESP and Exams. |  | Listen to the presentation, make notes on copy of slides and ask questions for clarification if necessary. Follow the explanation, offer answers about session 3 learning activities. Offer answers to pre-session task. Make notes on conservation of momentum. Collect the marked answer and make notes, as necessary. Offer answers, participate in the discussion, and make notes. |  |  | PowerPoint soft copy and hard copy: Section 01, slides 133 to 139 Glossary of terms Video with the answer to the pre-session task: (24) Rocket Propulsion - Due to Conservation of Momentum ¦ Physics with Professor Matt Anderson ¦ M9-03 – YouTube *Please note you may need to verify your age to watch this link | Probing questions |  |
| 60 mins | Teach momentum and impulse theories using slide 141 to 144 Distribute the experiment worksheet and ask learners to read the introduction. Explain design and modelling requirement of the trolleys of AOC. Probe questions for similar applications from the industry. Make two-member (max.3) groups and guide them to conduct the experiment on impulse and momentum following the experiment worksheet. Explain the steps to use the online experimenting tool. Facilitate the learner groups to locate the link and understand the steps. Mentor learner groups as they conduct the experiment. Review effective recording of results in table A. |  | Follow the explanation and make notes. Study the experiment worksheet, participate in the discussion and make notes. Get into allocated group and select a project to conduct a case study. Follow the explanation, locate resources, and start the experiment. Delegate responsibilities among team members, follow the steps outlined in the method section and fill in table A, as they obtain the results. Answer three true/false questions and participate in the discussion. |  |  | PowerPoint soft copy and hard copy: Section 02, slides 141 to 146 Experiment worksheet soft copy and hard copy Calculator Glossary of terms Online experiment link: Physics Simulation: Collisions (physicsclassroom.com) | Probing questions Experiment results table completion Three true/false assessment questions |  |
| 15 mins | Break |  |  |  |  |  |  |  |
| 40 mins |  | Review the table A of the experiment worksheet completed by learners. Ask the learner groups to complete table B by entering the calculation and the answers of the final velocity, impulse and force using the equations taught for each case. Mentor learner groups as they work on calculations. Probe questions to explain precautions that could be used to reduce the reaction force as a result of the collision and suitable modifications to avoid collision between two trolleys. Collate industrial examples for similar design situations on whiteboard. Guide learners to answer the discussion questions individually. Ask learners to answer three true/false questions in slide 146, probe questions for deeper understanding. |  | Get the table A reviewed by the teacher. Follow the guidance and complete the table B by including all the required calculation in the space provided. Ask for clarifications if necessary. Offer answers, participate in the discussion and make notes. Write the answers to discussion questions, individually. Answer three true/false questions and participate in the discussion. | PowerPoint soft copy and hard copy: Section 02, slides 141 to 146 Own notes A4 sheets/pencil/pen Experiment worksheet soft copy and hard copy. Calculator Whiteboard |  |  | Probing questions Experiment results table completion three true/false assessment questions |

| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| --- | --- | --- | --- | --- |
| 20 mins | Explain the problems 1, 2 and 3 on slides 148, 149 and 151, provide time to calculate answers and mentor the learners as they work. Make sure to show the video on slide 148, after explaining the problem 1. Invite a learner to write the answer for each case on whiteboard and provide feedback for the answers received. Do the workings on whiteboard if necessary. Lead learners to self-assess their work. *This exercise contributes to development of skills required by ESP. Ask learners to answer three true/false questions in slide 21, probe questions for deeper understanding. | Follow the explanation and make notes. Calculate the required quantities for the given problems. Watch the video, seek clarification if necessary. Learners offer answers as directed by the teacher, make notes on improving the quality of the answer. Mark the answer using the teacher’s workings. Answer three true/false questions and participate in the discussion. | PowerPoint soft copy and hard copy: Section 03, slides 148 to 152 Video link Q1: Movax Pile Driving Equipment ¦ Piling Hammers for sale from Premier Rock Machinery - IndustrySearch Australia Own notes Pencil/pen, calculator Whiteboard to show the workings. | Probing questions. three true/false assessment questions |
| 15 mins | Plenary Elicit learning from the learners and scribe on board. Explain the next session’s aim and relation to PBL and how today’s sessions content is linked. Share the pre-session task on slide 155 and the pre-session video link. Provide time to complete self-assessment sheet. | Learners offer recap of main learning in the session. Note the pre-session video link and task. Follow the explanation and make notes. Complete the self-assessment sheet. | Video: (42) Workplace safety - Overhead cranes and hazard zones – YouTube *Please note you may need to verify your age to watch this link Pre-session task Self-assessment sheet | Elicitation Self-Assessment |

This programme is funded by

the Department for Education

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Funded by

Produced by

Session 5 – worksheet

Impulse and momentum experiment

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 5 worksheet

Scenario: ABC is an automobile manufacturing company that is considering designing and building a double girder automated overhead crane (AOC) system (Figure 01) to assist their car manufacturing process.

Figure 01: Double girder AOC system with two trolleys.

Figure 02: Modelling of two trolley system with two carts.

As part of the design process, the design engineers are analysing the trolley movements. The aim of the trolleys modelling process is to obtain suitable speed and acceleration value ranges applicable for safe operation of the trolleys.

Safety of the trolley movement is to be evaluated in two conditions.

- As the load is lifted free of the floor it will attempt to centre itself under the hoist, causing the load to rapidly swing in a horizontal arc (Figure 03). This pendulum effect can cause serious injury to personnel or damage to other equipment in the area.

- The possibility of a collision between two trolleys when in operation.

V

Figure 03: Crane side pulling because of the pendulum effect.

Safe operation velocities for trolleys would then be recommended by the design engineers in the operation manual of the double girder AOC.

The velocity analysis to avoid crane side pulling shall be done using the energy conservation principles, which will be taught in session 6.

Different cases to model the velocity and impulse analysis shall be done using today’s experiment.

## Resources

Physics Simulation: Collisions (physicsclassroom.com)

Follow the instructions given on the experiment handout and complete the results table.

Method

- Set the initial velocity of the red and blue cart to the given values in the table A each for each case detailed in the table.

- Set the mass of the red and blue cart to the given values in the table A each for each case.

- Select the correct collision type from the three green buttons given (elastic, inelastic or explosion).

- Click on the start button at the top right corner.

- Note down the final velocity values of red and blue cart just after the collision.

- Click on the reset button at the top left corner and follow the steps 1 to 6.

## Results

### Table A: Results from the experiment

| Case no. | Collision type | Red cart |  |  | Blue cart |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mass (kg) | Initial velocity (m/s) | Final velocity (m/s) | Mass (kg) | Initial velocity (m/s) | Final velocity (m/s) |
| a | Elastic | 1 | 5 |  | 1 | −5 |  |
| b | Elastic | 1 | 5 |  | 3 | −5 |  |
| c | Elastic | 1 | 10 |  | 2 | 1 |  |
| d | Elastic | 1 | 10 |  | 3 | 0 |  |
| e | Inelastic | 1 | 5 |  | 1 | −5 |  |
| f | Inelastic | 1 | 5 |  | 3 | −5 |  |
| g | Inelastic | 1 | 10 |  | 2 | 1 |  |
| h | Inelastic | 1 | 10 |  | 3 | 0 |  |
| i | Explosion | 1 | 0 |  | 1 | 0 |  |
| j | Explosion | 1 | 0 |  | 3 | 0 |  |

## Calculations

Provide the theoretical calculations for each of the above case in table B.

Calculate the impulse and the reaction force on red and blue cart for each of the below cases.

Consider the contact time is 0.2 s. Note that the reaction force on red and blue carts are equal in each case.

### Table B: Results from the theoretical calculations

| Case no. | Collision type | Red cart |  |  | Blue cart |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mass (kg) | Initial velocity (m/s) | Final velocity (m/s) | Mass (kg) | Initial velocity (m/s) | Final velocity (m/s) |
| a | Elastic | 1 | 5 |  | 1 | −5 |  |
| b | Elastic | 1 | 5 |  | 3 | −5 |  |
| c | Elastic | 1 | 10 |  | 2 | 1 |  |

| Case no. | Collision type | Red cart |  |  | Blue cart |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mass (kg) | Initial velocity (m/s) | Final velocity (m/s) | Mass (kg) | Initial velocity (m/s) | Final velocity (m/s) |
| d | Elastic | 1 | 10 |  | 3 | 0 |  |
| e | Inelastic | 1 | 5 |  | 1 | −5 |  |
| f | Inelastic | 1 | 5 |  | 3 | −5 |  |

| Case no. | Collision type | Red cart |  |  | Blue cart |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mass (kg) | Initial velocity (m/s) | Final velocity (m/s) | Mass (kg) | Initial velocity (m/s) | Final velocity (m/s) |
| g | Inelastic | 1 | 10 |  | 2 | 1 |  |
| h | Inelastic | 1 | 10 |  | 3 | 0 |  |
| i | Explosion | 1 | 0 |  | 1 | 0 |  |
| j | Explosion | 1 | 0 |  | 3 | 0 |  |

## Discussion

- Compare your velocity values from experiments entered in table A, with the theoretically analysed values entered in table B for each experimental case a to j.

- Explain precautions that could be used to reduce the reaction force as a result of the collision and suitable modifications to avoid collision between two trolleys.

- Provide five industrial applications where the momentum principles would be used.

## Session 5

## Self-assessment sheet

### Please RAG rate your confidence (Red= I am not confident, Amber = I need more practice, Green = I am confident)

| Learning objectives | R | A | G |
| --- | --- | --- | --- |
| - I understand how to calculate impulse for a given system. | ☐ | ☐ | ☐ |
| - I understand how to calculate momentum for a given system. | ☐ | ☐ | ☐ |
| - I understand how to calculate velocity after a collision for both elastic and inelastic collisions. | ☐ | ☐ | ☐ |
| - I understand the application of momentum in PBL project scenario. | ☐ | ☐ | ☐ |
| - I know how to access the pre-session video link and understand the task in hand. | ☐ | ☐ | ☐ |

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Introduction to engineering mechanics

Lesson plan six

| Session number: 6 of 7 | Engineering mechanics: Application of Newton’s laws |
| --- | --- |
| City & Guilds ref | 8730 |
| Tutor notes and session rationale This session is focused on the calculation of energy and power and their applications in the engineering and manufacturing industry. The session starts with an introduction to the lesson and a recap of session 5, which is then assessed using Geoff Petty’s true/false formative assessment activity. Review of pre-session task is vital in all these sessions, as it establishes the groundwork for the rest of the session. This helps to establish a continuum of self-directed learning and formative assessment needed in examination-based courses. This session assumes that the completed pre-session task and the session 5 self-evaluation have been reviewed by the teacher before session 6. This session uses the same problem-based learning (PBL) project scenario that was used in sessions 1 to 5. Learners’ prior knowledge: Teaching of this session starts with eliciting/pre-teaching activities to ensure all learners’ knowledge of the following is sound and has not become dormant. For more details, refer to lesson plan 1. Pre-session task submission: Facilitate submission of either soft copy or hard copy of completed pre-session tasks before the session. Maintain a submission record. |  |  |
| Embedded skills development |  |  |
| Science | Application, knowledge and understanding of Newton’s three laws of motion, including appropriate calculations. (5.6, 7.1 and 7.2) *City & Guilds specification numbers Understanding and skills of units of measurement used in engineering. (5.1)* Understanding and skills in vectors applications. (5.2)* Understand scientific methods and approaches to scientific inquiry and research. (5.3)* |
| Maths1 | Interpreting and representing with mathematical diagrams. Using rules and formulae. Communicating using mathematics. |
| English2 | Summarise, synthesise and present information and ideas. Take part in/lead discussions. Develop note taking skills. |
| Digital3 | Use digital technology and media effectively. |
| Employability | The problem-based learning (PBL) project includes opportunities to develop problem-solving skills, analytical thinking and teamworking skills that are useful to work in the industry. |
| Health and safety | Highlight the health and safety aspects in relation to design of the trolley system of an overhead crane system. |
| Employer-set project | An introduction to how the activities and the learning on this course can be used in an employer-set project. |
| Examinations | The science concepts shall be assessed as part of the core content for the T Level in Engineering and Manufacturing. |

| Outcomes – by the end of the session learners will: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Apply science concepts to a range of topics found in the industrial environments in the Engineering and Manufacturing route. Evaluate health safety of cranes and traversing trolleys on overhead cranes. Demonstrate the ability to: - understand the terms and application of tractive force and braking force - understand different types of mechanical energy - calculate mechanical energy for given systems - calculate power in mechanical systems. |  |  |  |  |
| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| 30 mins | Introduce the session objectives. Explain how the sessions are linked and contributing to the problem-based learning (PBL) project on automated overhead crane (AOC) design project. Guide learners to make notes on their copy of the slides. Ask learners to offer examples of activities undertaken in session 5 under each objective listed in slide 161. Ask learners to answer three true/false questions on slide 162, probe questions for deeper understanding. Act as scribe for pre-session task answers and review. Use slide 164 or whiteboard to summarise answers. Probe questions for deeper understanding. Emphasise use of sensors such as anti-collision sensor, use of signals and importance of safety training. Explain how the momentum principles shall be used to model any collisions, F=ma can be used to model the angle of swing of the weight and the swing distance can be calculated for any sudden stops of the trolley using energy conservation that is to be discussed in this session. Probe questions on applications of knowledge of energy principles in the industry. *This exercise contributes to development of skills required by ESP. | Listen to the presentation, make notes on copy of slides and ask questions for clarification if necessary. Follow the explanation, offer answers about session 5 learning activities. Answer three true/false questions and participate in the discussion. Offer answers to presession task. Make notes on risks and safety precautions. Participate in the discussion, listen to the explanations and make notes. Offer answers and make notes. | PowerPoint soft copy and hard copy: Section 01, slides 159 to 164 Glossary of terms | three true false assessment questions Probing questions |
| 20 mins | Teach equations of motion using slide 166. Guide learners to access the online quiz available on slide 167. Provide adequate time and guidance to complete the questions on BBC Bitesize website. Model the workings on whiteboard to show the steps to achieve the answer. Guide to self-assess. Learners will have consolidated this knowledge as homework and/or in future lessons outside of this seven-week course. *This exercise contributes to development of skills required by Exams. | Follow the explanation and make notes. Access the quiz links on slide 167 and complete with the guidance of teacher. Ask for clarifications if necessary. Self-assess using the answer on the website and whiteboard. | PowerPoint soft copy and hard copy: Section 02, slides 166 to 170 Own notes Calculator Glossary of terms Quizzes: https://www.bbc.co.uk/bitesize/guides/zxd9w6f/revision/7 | Online quiz |
| 40 mins | Teach tractive force and breaking force using slide 168 and 169. Probe questions on suitable industrial applications. Form two-member groups (max. three) and facilitate the group activity on stopping distance using slide 170. Distribute the simulation activity sheet and guide learners to follow the steps to gather required data. Guide to complete the activity sheet table. Facilitate accessing the online quiz and guide to self-assess. Learners will have consolidated this knowledge as homework and/or in future lessons outside of this seven-week course. *This exercise contributes to development of skills required by Exams. | Follow the explanation, answer questions and make notes. Get into groups and obtain access to experiment website. Follow the activity sheet guidance and obtain data. Complete the results table on activity sheet. Access the concept checker quiz online and self-assess knowledge on stopping distance. | PowerPoint soft copy and hard copy: Section 02, slides 168 to 170 Simulation activity sheet soft copy and hard copy: StoppingDistance (physicsclassroom.com) Own notes Calculator Glossary of terms Simulation website: Physics Simulation: Stopping Distance (physicsclassroom.com) Quiz: Stopping Distance Concept Checker (physicsclassroom.com) | Probing questions Online quiz |
| 15 mins | Break |  |  |  |

| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| --- | --- | --- | --- | --- |
| 30 mins | Teach energy and power equations using slide 172 to 178. Elicit possible applications from the industry and day-to-day life. Guide learners to access the online quizzes available on slide 175 and 179. Provide adequate time and guidance to complete the quiz with full marks. Do any required workings on whiteboard to show the steps to achieve the answer. Learners will have consolidated this knowledge as homework and/or in future lessons outside of this seven-week course. *This exercise contributes to development of skills required by Exams. | Follow the explanation and make notes. Participate in the discussion. Access the quiz links on slide 175 and 179 and complete with the guidance of teacher. Ask for clarifications if necessary. Re-do the quiz until learner achieves full marks. | PowerPoint soft copy and hard copy: Section 03, slides 172 to 179 Own notes Calculator Glossary of terms Quizzes: Changes in energy stores test questions – AQA – GCSE Physics (Single Science) Revision – BBC Bitesize Work, power and efficiency test questions – AQA Trilogy – GCSE Combined Science Revision – BBC Bitesize | Online quizzes |

| 30 mins | Guide learners to access the pendulum lab online simulation and study the energy variation. Provide time to do problem 1. Model problem 1 on whiteboard. Guide to self-assess the answer using slide 183. Guide learners to calculate the horizontal distance of the mass, in slide 184. Provide the workings on board and guide to self-assess. Guide learners to access the mass and springs lab online simulation and study the energy variation. Provide time to do problem 2. Probe questions and explain the answer. *This exercise contributes to development of skills required by ESP and Exams. | Access the pendulum lab online simulation link and study as guided by teacher. Answer problem 1. Study how the answer is done by teacher, ask for clarification, make notes. Self-assess the answer. Calculate the side pull as explained in slide 184. Self-assess the answer and make notes. Access the pendulum lab online simulation link and study as guided by teacher. Answer problem 2. Ask for clarifications and make notes. | PowerPoint soft copy and hard copy: Section 04, slides 181 to 186 Own notes A4 sheets/pencil/pen Online lab: Pendulum Lab‬ (colorado.edu) Online lab: Masses and Springs‬ (colorado.edu) Calculator Whiteboard | Probing questions. Independent problem solving. |
| --- | --- | --- | --- | --- |
| 15 mins | Plenary Elicit learning from the learners and scribe on board. Explain the next session’s aim and relation to PBL and how today’s sessions content will be applied in the context of exam, ESP and industry placement/future work. Share the pre-session task on slide 189 and the pre-session video link. Provide time to complete self-assessment sheet. | Learners offer recap of main learning in the session. Note the pre-session video link and task. Follow the explanation and make notes. Complete the self-assessment sheet. | PowerPoint soft copy and hard copy: Section 04, slides 188 to 191 Videos: A guide to the energy of the Earth – Joshua M. Sneideman – YouTube Renewable Energy 101 ¦ National Geographic – YouTube *Please note you may need to verify your age to watch this link Pre-session task Self-assessment sheet | Elicitation Self-Assessment |

This programme is funded by

the Department for Education

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Funded by

Produced by

Session 6 – self-assessment sheet

Application of Newton’s laws

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 6

## Self-assessment sheet

### Please RAG rate your confidence (Red= I am not confident, Amber = I need more practice, Green = I am confident)

| Learning objectives | R | A | G |
| --- | --- | --- | --- |
| - I know how to calculate kinetic energy. | ☐ | ☐ | ☐ |
| - I know how to calculate gravitational and elastic potential energy. | ☐ | ☐ | ☐ |
| - I know how to calculate mechanical energy. | ☐ | ☐ | ☐ |
| - I know how to calculate power. | ☐ | ☐ | ☐ |
| - I understand the use of work done calculation to industrial applications. | ☐ | ☐ | ☐ |
| - I understand the application of Newton’s laws in double girder AOC and similar systems. | ☐ | ☐ | ☐ |
| - I know how to access the pre-session video link and the task. | ☐ | ☐ | ☐ |

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Lesson plan seven

Introduction to engineering mechanics

| Session number: 7 of 7 | Engineering mechanics: Application of Newton’s laws |
| --- | --- |
| City & Guilds ref | 8730 |
| Tutor notes and session rationale This session is focused on calculation of rotational kinetic energy, its applications and the need for renewable energy sources in the engineering and manufacturing industry. The session starts with an introduction to the lesson and a recap of session 6, which is then assessed using a calculation activity from the content in session 6. Review of the pre-session task is vital in all these sessions, as it establishes the groundwork for the rest of the session. This helps to establish a continuum of self-directed learning and formative assessment needed in examination-based courses. This session assumes that the completed pre-session task and the session 6 self-evaluation have been reviewed by the teacher before session 7. This session uses the same problem-based learning (PBL) project scenario that was used in sessions 1 to 6. Learners’ prior knowledge: Teaching of this session starts with eliciting/pre-teaching activities to ensure all learners’ knowledge of the following is sound and has not become dormant. For more details, refer to lesson plan 1. Pre-session task submission: Facilitate submission of completed pre-session tasks, as either soft copy or hard copy, before the session. Maintain a submission record. |  |  |
| Embedded skills development |  |  |
| Science | Application, knowledge and understanding of Newton’s three laws of motion, including appropriate calculations. (5.6, 7.1 and 7.2) *City & Guilds specification numbers Understanding and skills of units of measurement used in engineering. (5.1)* Understanding and skills in vectors applications. (5.2)* Understand scientific methods and approaches to scientific inquiry and research. (5.3)* |
| Maths1 | Interpreting and representing with mathematical diagrams. Using rules and formulae. Communicating using mathematics. |
| English2 | Summarise, synthesise and present information and ideas. Take part in/lead discussions. Develop note taking skills. |
| Digital3 | Use digital technology and media effectively. Communicate and collaborate. Be safe and responsible online. |
| Employability | The problem-based learning (PBL) project includes opportunities to develop problem-solving skills, analytical thinking and teamworking skills that are useful to work in the industry. Presentation skills development is also a key aspect improved from the session. |
| Health and safety | Highlight the health and safety aspects in relation to energy generation methods. |
| Employer-set project | An introduction to how the activities and the learning on this course can be used in an employer-set project. |
| Examinations | Practice exam skills through an in-class exam to assess principles of science as part of the core content for the T Level in Engineering and Manufacturing. |

| Outcomes – by the end of the session learners will: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Apply science concepts to a range of topics found in the industrial environments in the Engineering and Manufacturing route. Evaluate health safety implications of different energy sources used in the world. Demonstrate the ability to: - understand rotational kinetic energy and moment of inertia - identify types of power sources and practical applications (flywheels, springs, height, pressurised fluids) - understand the need for renewable energy sources and sustainable materials used to reduce energy requirement - practice exam skills required for the T Level qualification. |  |  |  |  |
| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| 30 mins | Introduce the session objectives. Guide learners to make notes on their copy of slides. Ask learners to offer examples of activities undertaken in session 4 under each objective listed in slide 195. Ask learners to answer the question on slide 196 and probe questions for deeper understanding. Act as scribe for pre-session task answers and review. Highlight the importance of renewable energy generation methods for a sustainable future by probing questions, using slide 197. Probe questions to look at the energy conversions involved in different energy generation methods and guide learners to answer the extended question from the pre-session task, available on slide 199. Explain the answer on whiteboard. Guide to self-assess the answers. *This exercise contributes to development of skills required by ESP. | Listen to the presentation, make notes on copy of slides and ask questions for clarification if necessary. Follow the explanation, offer answers about session 3 learning activities. Answer the questions and participate in the discussion. Offer answers to pre-session task. Make notes on conservation of momentum. Offer answers, participate in the discussion and make notes. Answer the question on slide 199. Self-assess answers and make notes, as necessary. | PowerPoint soft copy and hard copy: Section 01, Slides 193 to 199 Glossary of terms | One calculation question from session 6 Probing questions |
| 30 mins | Explain how the rotation kinetic energy is linked to power generation to generate curiosity to learn theories. Ask pre-video question and discuss answers. Play the video and probe questions for deeper understanding. Probe question on the rotational objects used to store energy, play the two videos and probe questions for deeper understanding. Discuss the flywheel applications from day-to-day life and engineering industry. *This exercise contributes to development of skills required by ESP and exams. | Follow the explanation and make notes. Answer pre-video question, watch the video, participate in the discussion and make notes. Offer answers, watch the video, participate in the discussion and make notes. Participate in the discussion, watch the video and make notes. | PowerPoint soft copy and hard copy: Section 02, slides 201 to 203 Glossary of terms Videos: https://www.youtube.com/watch?v=lNx0yPdl960 https://www.youtube.com/watch?v=CHQOctEvtTY https://www.youtube.com/watch?v=W9fPGXyvrVM https://www.youtube.com/watch?v=EMazLuxpzxE *Please note you may need to verify your age to watch these links Laptops/ Computers A4 sheets/ pens/ pencils | Probing questions |
| 30 mins | Probe question 1 on slide 204 on energy storing methods, discuss about flywheels, springs, height, pressurised fluids. Form groups of two members (max 3) and ask to answer the Q2 and Q3 on slide 204, by conducting a short online survey. Ask each group to present their findings to the class and probe questions for deeper understanding. Emphasise the fact that electricity cannot be stored in massive amounts before distribution. Highlight the primary energy storing mechanism such as water stored by the dams, in hydropower. *This exercise contributes to development of skills required by ESP and exams. | Offer answers, participate in the discussion and make notes. Get into groups to study and answer Q2 and Q3. Present findings to the class and participate in the discussion. Make notes. Follow the explanation and make notes. Ask for clarifications if necessary. | PowerPoint soft copy and hard copy: Section 02 Slide 204 Glossary of terms Laptops/ Computers A4 sheets/ pens/ pencils | Probing questions Group presentation Q/A |
| 15 mins | Break |  |  |  |

| Time | Tutor activity | Learner activity | Resources and slide number | Assessment of learning |
| --- | --- | --- | --- | --- |
| 25 mins | Explain the problem detailed on slide 206 probe questions to revisit knowledge from session 1 to 7 relevant for the question. Emphasise that the questions are linked to worksheet 1 questions where the forces were calculated for similar scenarios. Provide time to calculate answers for part 1 on slide 208 and mentor the learners as they work. Show the answer for Part 1 on slide 209, provide feedback for the answers done. Lead learners to self-assess their work. Provide time to calculate answers for part 2 on slides 210 and 211. Mentor the learners as they work. Show the answer for on slides 212 to 216, provide feedback for the answers done. Lead learners to self-assess their work. *This exercise contributes to development of skills required by ESP and exams. | Follow the explanation and make notes. Offer answers and participate in the discussion. Calculate the required quantities for the given problems. Seek clarification if necessary. Self-assess using slide 209 and guidance from teacher. Learners offer answers as directed by the teacher, make notes on improving the quality of the answer. Mark the answer using the teacher’s workings. Ask for clarifications. | PowerPoint soft copy and hard copy: Section 03, slides 206 to 216 Glossary of terms Laptops/ Computers A4 sheets/ pens/ pencils Whiteboard | Probing questions. Independent problem solving. |
| 65 mins | Distribute the exam paper on the application of Newton’s laws. Facilitate learners to complete the one-hour exam. Collect the exam papers. **The exam papers shall be marked using the marking scheme provided. Individual feedback on exam performance shall be given upon completing the marking, preferably within two weeks of finishing this exam. *This exercise contributes to development of skills required by exams. | Obtain the exam paper and answer for one hour. Hand-over the exam papers. | Exam paper hard copy Own notes Pencil/pen, calculator Glossary of terms | Exam questions solving. |
| 15 mins | Plenary Provide the further reading links on renewable energy. Elicit learning from the learners and scribe on board. Explain the how the seven sessions on Newton’s laws applications will help their tasks in the industry. Take the PBL as an example. Provide time to complete self-assessment sheet. | Obtain access to the further reading links. Learners offer recap of main learning in the session and the whole course. Follow the explanation and make notes. Complete the self-assessment sheet. | PowerPoint soft copy and hard copy: Section 04 and 05, slides 219 to 223 Self-assessment sheet Further reading links: https://www.nhm.ac.uk/discover/renewable-energy.html https://ww3.rics.org/uk/en/modus/natural-environment/renewables/the-future-of-sustainable-building-materials.html https://eandt.theiet.org/content/articles/2021/10/the-green-house-effect-building-greener-to-tackle-climate-change | Elicitation Self-Assessment |

This programme is funded by

the Department for Education

Produced by

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Session 7 – exam paper – one hour

Application of Newton’s laws

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 7 – exam – one hour

- The design engineers of ABC company are working on an automated ramp system for their manufacturing workshop. As part of the design process, the following case is considered.

A hollow box will be used to transport the tools to an elevated workshop area as illustrated in the diagram below (figure 01). The box will be pulled along an inclined ramp (20° to horizontal) using a pully-cable system. Both the box and the ramp surface are to be made with a frictionless material. The maximum mass of the box filled with the tools will be approximately 125 kg.

The tasks below detail the steps associated with calculation of tension of the rope when the box moves up on the ramp in a constant velocity. This will be used to recommend a suitable diameter for the cable.

- Draw the FBD for the inclined ramp.

- Calculate the value of the tension force when the box is moving at a constant velocity.

- Calculate the value of the normal force.

- Calculate the cable tension, if the kinetic friction coefficient of the surface is 0.42 and the box is moving at constant velocity.

- If the operating velocity is 0.45 m/s, calculate the work done by the box per second.

Figure 01: Ramp and box arrangement.

(3 × 5 = 15 marks)

- ABC is an automobile manufacturing company that is considering designing and building a double girder automated overhead crane (AOC) system (figure 02) to assist their car manufacturing process.

Figure 02: Double girder AOC system.

For the safety of the traversing trolley mechanism, engineers are planning to install sensors in both side of the trolleys, as shown in the diagram below (figure 03).

Figure 03: Traversing trolley with the anti-collision signals.

Design details:

- Maximum speed the traversing trolleys can move is 1.2 m/s.

- Mass of the trolley is 15 kg.

- Maximum mass attached to the trolley is 1 tonne.

- Maximum breaking force that could be applied on a traversing trolley is 2kN.

For safety, both the trolleys will be applying breaking force when the anti-collision sensors are activated. The delay time to apply breaking force is 0.2 s.

Calculate the minimum gap that should activate the anti-collision sensors of the trolleys using conservation energy principle.

(10 marks)

- A pile-driver is used to install foundation piles for a tall building (figure 04). The mass of the hammer us 1000 kg and it is lifted to 3.5 m above the pile head and then allows to drop. The force exerted on the pile head because of the impulse is calculated as 1.5 MN.

Force on pile = 1.8 MN

Figure 04: Slide 17 from session 5

Calculate the approximate distance travelled by the pile through the ground, assuming the friction applied on the pile by the soil is 1035 kN.

(10 marks)

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Session 7 – exam paper marking scheme

Application of Newton’s laws

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 7 – exam paper marking scheme

1.

- Draw the FBD for the inclined ramp.

20˚

Diagram with three forces - 3

Calculate the value of the tension force when the box is moving at a constant velocity.

Method – 2

Answer – 1

Units must be accurate.

- Calculate the value of the normal force.

Method – 2

Answer – 1

Units must be accurate.

20˚

Calculate the cable tension, if the kinetic friction coefficient of the surface is 0.42 and the box is moving at constant velocity.

Method – 2

Answer – 1

Units must be accurate.

- If the operating velocity is 0.45 m/s, calculate the work done by the box per second.

Method – 2

Answer – 1

Units must be accurate.

Work done per second 				= F×dt J/s

= F×v W

Force to move the trolley in constant velocity 	= 903.4 N

Operating velocity				  	= 0.45 m/s

Hence, work done by the trolley per second 	= 903.4×0.45 W

= 406.53 W

≅0.41 kW

2.	For the safety of the traversing trolley mechanism, engineers are planning to install sensors in both side of the trolleys, as shown in the diagram below.

x

x

d

d

Design details:

- Maximum speed the traversing trolleys can move is 1.2 m/s.

- Mass of the trolley is 15 kg.

- Maximum mass attached to the trolley is 1 tonne.

- Maximum breaking force that could be applied on a traversing trolley is 2kN.

For safety, both the trolleys will be applying the breaking force when the anti-collision sensors are activated. The delay time to apply breaking force is 0.2 s.

Calculate the minimum gap that should activate the anti-collision sensors of the trolleys using conservation of energy principle.

Assume the trolleys are moving at maximum speed and the sensors got activated when the trolleys are (2x+2d) distance apart, where d is the distance travelled by one trolley during the delay time and x is the distance travelled by one trolley during the breaking time.

Calculation of the breaking distance of one trolley:

Kinetic energy of both the trolley and mass when the breaking is applied

= 12mv2

= 121015×1.22 = 730.8 J

Method – 2

Answer – 1

Units must be accurate.

Maximum breaking force 				= 2000 N

Breaking distance 					= x m

Work done by the breaking force 			= F×x=2000×x

From conservation of energy principle,

Method – 2

Answer – 1

Units must be accurate.

Work done by the breaking force = Kinetic energy of the system when breaking was initiated

2000×x=730.8
x=730.82000m
x=0.3654 m
x=36.54 cm

Method – 2

Answer – 1

Units must be accurate.

Calculation of the distance travelled by one trolley during delay time:

S=ut+12at2
d=1.2×0.2+0
d=0.24 m
d=24 cm

Sensor activation gap = 2d+2x=(2×24+2×36.54) cm

2d+2x=121.08 cm

3.	A pile-driver is used to install foundation piles for a tall building. The mass of the hammer us 1000 kg and it is lifted to 3.5 m above the pile head and then allows to drop. The force exerted on the pile head because of the impulse is calculated as 1.5 MN.

Method – 2

Answer – 1

Units must be accurate.

Calculate the approximate distance travelled by the pile, assuming the friction applied on the pile is 1,035 kN.

Friction force  			= 1,035 kN

Applied force  			= 1.5 MN

Resultant force acting on pile 	= (1.5 x 1000 – 1,035 ) kN

= 465 kN

= 465,000 N

Method – 2

Answer – 1

Units must be accurate.

The potential energy of the pile-driver hammer  = mgh

=1000×9.81×3.5 J

= 34,335 J

Assumption - 1

Assuming the potential energy of the pile-driver hammer is fully converted as the work done on the pile.

Work done on the pile =                      F×x=465,000×x

Method – 2

Answer – 1

Units must be accurate.

465,000×x=34,335
                   x=34,335  465,000m
                   x=0.074 m
                 x=7.4 cm

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

Session 7 – self-assessment sheet

Application of Newton’s laws

etfoundation.co.uk

Introduction to engineering mechanics

# Introduction to engineering mechanics

## Session 7

## Self-assessment sheet

### Please RAG rate your confidence (Red= I am not confident, Amber = I need more practice, Green = I am confident)

| Learning objectives | R | A | G |
| --- | --- | --- | --- |
| - I understand rotational kinetic energy and moment of inertia. | ☐ | ☐ | ☐ |
| - I know the types of power sources and practical applications such as flywheels. | ☐ | ☐ | ☐ |
| - I Understand the need for renewable energy sources. | ☐ | ☐ | ☐ |
| - I understand the requirement of sustainable materials to reduce energy use. | ☐ | ☐ | ☐ |
| - I understand how Newton’s laws are applied to industrial context. | ☐ | ☐ | ☐ |
| - I understand working on exam style questions on application of principles of science. | ☐ | ☐ | ☐ |

This programme is funded by

the Department for Education

Funded by

Hugh Baird College has produced this resource on behalf of the Education and Training Foundation

Produced by

theiet.org/education