T LEVEL RESOURCE IMPROVEMENT PROJECTS

Pan Leeds Health T Level

Demonstrating clinical skills - Health T Level teaching and learning resource

ET-FOUNDATION.CO.UK

# Synopsis

| ## Clinical skills |  |
| --- | --- |
| Objectives: | - Demonstrate the ability to carry out clinical skills for individuals including clinical assessments and reporting findings. - Taking physiological measurements using the correct equipment and procedure to ensure accuracy, precision and any sampling errors are avoided. - Understand The relevance of current guidelines, standards, policies and frameworks, set by government, regulatory bodies and delivery partners to ensure core values of care are adhered to when assisting the adult nursing team with clinical skills. |
| Curriculum component: | - S1.18 - K1.2 |
| Core skills: | - CS5.1, CS5.2: Researching - CS1.1, CS1.2: Demonstrate person-centred care skills - CS4.1, CS4.2: Reflective Evaluation - CS2.1, CS2.2: Communication |

## Health T Level resource (TRIP) - Assist the adult nursing team with clinical skills

Course components considered in this resource:

S1.18 - Demonstrate the ability to carry out clinical skills for individuals including clinical assessments and reporting findings. Taking the following physiological measurements using the correct equipment and procedure to ensure accuracy, precision and any sampling errors are avoided.

K1.2 The relevance of current guidelines, standards, policies, and frameworks, set by government, regulatory bodies and delivery partners to ensure core values of care are adhered to when assisting the adult nursing team with clinical skills:

- Department of Health and Social Care (DHSC)

- Nursing and Midwifery Council (NMC)

- Care Quality Commission (CQC)

- Skills for Care (SfC)

- Health and Social Care Act 2012

- Care Act 2014

- NHS constitution

- Nursing and Midwifery Council Code

- and Standards

- Care Certificate

Core skills demonstrated in this resource:

- CS5.1, CS5.2: Researching

- CS1.1, CS1.2: Demonstrate person-centred care skills

- CS4.1, CS4.2: Reflective Evaluation

- CS2.1, CS2.2: Communication

Activity

To develop a student’s understanding of the different observations undertaken with patients in a clinical setting, why they are done and to ensure they are completed correctly. To allow students to develop clinical skills in the following tasks before industry placement.

- Weight

- Height

- BMI

- Body temperature

- Blood pressure

- Respiration rate

- Heart rate

- Oxygen saturation

- Collection of urine and faecal specimens

## Weight

To ensure you take reliable measurements using body weight scales you must:

- Zero the scales before the patient steps onto them

- Ask the patient to remove any ‘heavy’ items from their pockets (key’s, wallets etc) and remove any heavy items of clothing or apparel (big jackets, shoes etc)

- When measuring weight – ask the patient to look straight ahead and stay still on the scales. Wait for the needle/digital screen to settle before recording the measurement

## Height

Ideally height measurements will be taken using a ‘drop down’ tape measure fixed at about two metres on a wall or a specific piece of measuring equipment as shown adjacent. A reliable measurement could be taken without this equipment by marking a point (top of a patient’s head) against a wall and measuring up to it.

When taking measurements of height, you must:

- Ask your patient to remove their shoes prior to taking the measurement

- Ask your patient to stand with their back to the wall and look directly forward. The back of their feet, calves, bottom, upper back, and the back of their head should all be in contact with the wall.  They should be positioned directly underneath the drop-down measuring device.

- Lower the measuring device until it rests gently on the top of your patient’s head and record the measurement

## BMI

Calculating BMI (body mass index) A very common method of measuring bodyweight as a risk factor (cardiovascular disease, diabetes etc) is BMI (Body Mass Index).  The calculation is based on comparing a person's weight against their height. It applies equally to men and women.

## The equation for BMI

BMI =         Weight (kg)

Height (m)2

The following table categorises people according to their BMI results:

BMI Categories

Underweight = <18.5

Normal = 18.5-24.9

Overweight = 25.0-29.9

Obesity = 30.0->40

# Summary

Measuring BMI is an inexpensive method which can be used to assess if a patient’s body weight compared to their height is healthy or not.  It can also be used to assess their weight as a risk factor for the development of diseases.  BMI is also easy to calculate, and the results are repeatable and consistent.

The following chart can be used as an alternative for calculating clients BMI, by finding their height and following it across until it meets the line of their weight.

## Body Temperature - How to Take a Temperature in Adult Nursing

Take an adult’s temperature by mouth, in the ear or under the armpit. The armpit method is less accurate and is normally only used if the person is extremely drowsy or not clear mentally, you will learn how to take a temperature in the ear.

### Tympanic method (in the ear) - The ear method is recommended for adults.

- Use a clean probe tip each time, and follow the manufacturer’s instructions carefully.

- Gently tug on the ear, pulling it back. This will help straighten the ear canal, and make a clear path inside the ear to the eardrum.

- Gently insert the thermometer until the ear canal is fully sealed off.

- Squeeze and hold down the button for one second.

- Remove the thermometer and read the temperature.

A normal temperature is between 35.5-37.5°C. Temperatures may vary throughout the day, rising as much as one degree in the morning and reaching a maximum during the late afternoon. Mild increases may be caused by exercising, too much clothing or bedding, taking a hot bath or being outside in hot weather.

## Blood Pressure

Blood pressure is a measure of the force that your heart uses to pump blood around your body. Blood pressure is measured in millimetres of mercury (mmHg) and is given as two figures:

- Systolic pressure – the pressure when your heart pushes blood out.

- Diastolic pressure – the pressure when your heart rests between beats.

For example, if your blood pressure is "140 over 90" or 140/90mmHg, it means you have a systolic

pressure of 140mmHg and a diastolic pressure of 90mmHg.

As a general guide:

- Ideal blood pressure is considered to be between 90/60mmHg and 120/80mmHg.

- High blood pressure is considered to be 140/90mmHg or higher.

- Low blood pressure is considered to be 90/60mmHg or lower.

## High blood pressure is often related to unhealthy lifestyle habits, such as smoking, drinking too much alcohol, being overweight and not exercising enough. Left untreated, high blood pressure can increase your risk of developing a number of serious long-term health conditions, such as coronary heart disease and kidney disease.

## Low blood pressure is less common. Some medicines can cause low blood pressure as a side effect. It can also be caused by a number of underlying conditions, including heart failure and dehydration.

For more information, please visit www.nhs.uk.

## How blood pressure is tested

A device called a sphygmomanometer will be used to measure your blood pressure. This usually consists of a stethoscope, arm cuff, pump, and dial, although automatic devices that use sensors and have a digital display are also commonly used nowadays. A manual blood pressure is often believed to be more accurate than a digital display in the nursing profession.

## Respiratory rate

A person's respiratory rate is the number of breaths you take per minute. The normal respiration rate for an adult at rest is 12 to 16 breaths per minute. A respiration rate under 12 or over 25 breaths per minute while resting is considered abnormal.

Respiration rates may increase with fever, illness, and other medical conditions. When checking respiration, it is important to also note whether a person has any difficulty breathing. If you identify an abnormal respiration rate you should notify a doctor immediately.

## Heart rate (BPM)

A normal resting heart rate should be 60–100 beats per minute, but it can vary from minute to minute. It can go up to 130–150 beats or higher per minute when you're exercising – that's normal because the body needs to pump more oxygen-rich blood around the body.

Your pulse is the rate at which your heart beats. Your pulse is usually called your heart rate, which is the number of times your heart beats each minute (bpm). But the rhythm and strength of the heartbeat can also be noted, as well as whether the blood vessel feels hard or soft. Changes in your heart rate or rhythm, a weak pulse, or a hard blood vessel may be caused by heart disease or another problem.

As your heart pumps blood through your body, you can feel a pulsing in some of the blood vessels close to the skin's surface, such as in your wrist, neck, or upper arm. Counting your pulse rate is a simple way to find out how fast your heart is beating.

To find your pulse in your wrist:

- Hold out one of your hands, with your palm facing upwards.

- Press the first (index) finger and middle finger of your other hand on the inside of your wrist, at the base of your thumb – don't use your thumb as it has its own pulse.

- Press your skin lightly until you can feel your pulse – if you can't find it, try pressing a little harder or move your fingers around.

- For more information, please visit www.nhs.uk.

When you find your pulse:

- Count the number of beats you feel for 60 seconds.

- Count the number for 30 seconds and multiply by two.

- This gives you your heart rate – the number of times your heart beats per minute (bpm).

- You can also check if your pulse is regular or irregular by feeling its rhythm for about 30 seconds. It's very common to have occasional irregular heartbeats, such as missed beats.

- If your pulse continues to be irregular, it can be a sign of atrial fibrillation – an irregular and often abnormally fast heart rate. This is more likely if you're 65 or older.

- For more information, please visit www.nhs.uk.

## Oxygen Saturation

Oxygen saturation refers to the amount of oxygen that's in your bloodstream. The body requires a specific amount of oxygen in your blood to function properly. The normal range of oxygen saturation for adults is 94 to 99 percent.

## How do you use an oxygen saturation monitor?

During a pulse oximetry reading, a small clamp-like device is placed on a finger. Small beams of light pass through the blood in the finger, measuring the amount of oxygen. It does this by measuring changes of light absorption in oxygenated or deoxygenated blood. This is a painless process.

## Collection of urine and faecal specimens

Specimen collection is the process of obtaining different types of bodily fluids for laboratory analysis or near-patient testing. It is often a first step in determining diagnosis and treatment.

When handling samples of any kind it is important that you follow infection control policies set out by your work or education setting. For your own protection you should always wear gloves, mask, apron and sometimes a visor if the fluids are likely to be contaminated or become airborne.

## Urine samples

A rapid urine test is the quickest way to test urine. This involves dipping a test strip with small square coloured fields on it into the urine sample for a few seconds. After that you have to wait a little for the result to appear. Depending on the concentration of the particular substance you are testing for, the fields on the test strip change colour. Then the resulting colours of the fields are compared with a colour table. The colour table can be found on the urine test package. It shows which colours indicate normal and abnormal values.

In a rapid urine test, a test strip is dipped into the urine and then compared with the coloured fields on the packaging.

Rapid urine tests are usually done as part of routine examinations – for example at a family doctor’s office, during antenatal visits, when being admitted to the hospital, or before surgery. They are also used in people who have acute symptoms like lower abdominal pain, stomach ache or back pain, frequent painful urination, or blood in their urine. Some people who have diabetes use this test to check their sugar levels.

Rapid urine tests can be done at doctor’s offices, in hospital, or at home. The test strips are available without a prescription at the pharmacy or on the internet. But they are not intended for self-diagnosis purposes, and should be used in consultation with a doctor.

For more information regarding the substances that rapid urine test can detect, please visit https://www.ncbi.nlm.nih.gov/books/NBK279350/.

Many substances are usually found only in certain amounts in urine, so higher or lower levels indicate a deviation from the norm.

## The following substances can be checked using a rapid urine test

- pH value (measure of the acidity of the urine. Normal values, depending on diet, range from about five to seven, where values under five are too acidic, and values over seven are not acidic enough).

- Protein (not usually found in urine).

- Sugar (glucose, not usually found in urine).

- Nitrite (not usually found in urine).

- Ketone (a metabolic product, not usually found in urine).

- Bilirubin (breakdown product of haemoglobin, not usually found in urine).

- Urobilinogen (breakdown product of bilirubin, not usually found in urine).

- Red blood cells (erythrocytes, not usually found in urine).

- White blood cells (leukocytes, not usually found in urine).

## What do the results tell us?

You can determine whether the results are within the normal range by using the package insert or the colour chart on the package. The pH value, for example, can be used to find out whether there is an increased risk of developing urinary stones. This is the case if the pH is too acidic (if the value is below five). A pH value over seven may be a sign of a bacterial urinary tract infection. Tests measuring other things can help detect other problems:

- High protein levels may be a sign of nephritis (a kidney inflammation).

- Ketones and sugar in urine are signs of high blood sugar.

- Leukocytes or nitrite may be a sign of a bacterial infection.

If the test results are abnormal, you need to see a doctor. As with all tests, the results of rapid urine tests are not always reliable. For this reason, it might be a good idea to have a more detailed test done in a lab.

## What is urinalysis?

Urinalysis is an initial basic test of the main features of the blood. It is often part of a routine examination and is frequently performed when people are admitted to hospital and before surgery. It can also be used to check abnormal results from a rapid urine test. Complete urinalysis is done in a laboratory. It usually involves three steps:

- Assessment of the colour, cloudiness, and concentration of the urine

- Examination of the chemical composition of the urine using a test strip

- Examination of the urine under a microscope to look for bacteria, cells and parts of cells

Urinalysis is used to find the cause of – or monitor – urinary tract infections, bleeding in the urinary system, or kidney or liver disease. It can also be used for diabetes, some diseases of the blood, and bladder stones.

## What does urinalysis test for?

In addition to the substances that can be detected using a rapid test, urinalysis can also test for the following:

- Creatinine (breakdown product of muscle metabolism, an indicator of kidney function).

- Bacteria (not usually found in urine).

- Urinary casts (cylindrical stuck-together structures that form in the renal tubules, not usually found in urine).

- Crystals (found if there are high concentrations of certain substances in the urine, not usually found in urine).

- Epithelial cells (cells that line the ureter, bladder and urethra).

## What do the results tell us?

With the help of these values, laboratories can also use urinalysis to detect signs of other problems:

- Cholesterol crystals can be caused by high levels of cholesterol in the urine, for example.

- Urinary casts are usually a sign of kidney disease, such as an inflammation of the renal pelvis.

Abnormal results can be discussed with a doctor and may be followed by more precise tests, such as a blood test.

## Faecal samples

### What do they test for in a stool sample?

The stool may be examined for hidden (occult) blood, fat, meat fibres, bile, white blood cells, and sugars called reducing substances. The pH of the stool also may be measured. A stool culture is done to find out if bacteria may be causing an infection.

Using the information above you should now be able to understand each clinical skill in more detail and following practical training be able to perform these tasks under observation.

This programme is funded by

the Department for Education

Leeds City College has produced this resource on behalf of the Education and Training Foundation

FUNDED BY

PRODUCED

BY